Table of Contents

# Chapter 0 — Why robots fly: the project and your toolkit

Welcome. Before we touch a single equation of motion, this chapter gets three things settled: why this project exists (the story it recreates), how we will learn (a specific method, not vibes), and where everything lives in the repository. It ends with the notation contract that every later chapter — and every line of code — obeys.

## After this chapter you can …

- Retell the “small is agile” scaling argument from Vijay Kumar’s TED talk and say which three ideas from the talk this repo implements from scratch.

- Navigate the repository confidently: say what each file in quadsim/ does, why SPEC.md exists, and where the images and GIFs in out/ come from.

- Run the test suite and any demo from the command line using the project’s virtual environment, and interpret a demo’s PASS/WARN output.

- Read and use the project’s notation — frames, e3, R, q, ω, f, τ — without flipping back to look things up.

- Diagnose the single most common tooling failure (wrong Python interpreter) in under a minute.

## The ECE bridge

Nothing in this chapter is truly foreign to you; here is the dictionary.

| New thing here | What you already know |
| --- | --- |
| The quadrotor as 13 states, 4 inputs | State-space: x=f(x,u), just nonlinear and with a quaternion inside the state vector |
| World frame vs body frame | I/Q demodulation: the body frame is a rotating reference frame, exactly like viewing a passband signal in the carrier’s frame at baseband |
| Quaternion q | The phasor ejθ one dimension up: a unit-norm object whose multiplication composes rotations, the way multiplying phasors adds phases |
| Control loop at dt=0.002 s (500 Hz) | Discrete-time DSP: sample rates, and the rule of thumb that the loop rate must comfortably exceed the plant’s bandwidth |
| SPEC.md | A datasheet / interface control document: register maps, pin conventions, signal polarities. You never argue with the datasheet mid-project |
| pytest suite | A testbench: automated stimulus + checked responses, run after every change, like regression tests on an RTL design |
| The venv .venv/ | A pinned toolchain: the same reason you lock one compiler/SDK version for an embedded target instead of “whatever is on the machine” |
| Thrust f and torque τ | The actuator drive signals — the analog outputs of your controller. On real hardware they become four PWM duty cycles (Chapter 7 territory, your home turf) |

## Core theory: the story, the argument, the method

### The talk and the three ideas

In 2012 Vijay Kumar showed a TED audience palm-sized quadrotors doing things that looked impossible: snapping through thrown hoops, flying aggressive figure-eights, and — the finale — nine robots morphing between formations in tight airspace. The magic decomposed into three concrete pieces of engineering, and this repository implements all three from scratch on plain numpy/scipy/matplotlib:

- Minimum-snap trajectories (Mellinger & Kumar, 2011) — planning motion so smooth that the motors are never asked for violent commands. This is why the flight looks graceful.

- Geometric SE(3) control (Lee, Leok & McClamroch, 2010) — feedback done directly on the rotation group, with no small-angle approximation, so large tilts don’t break the math. This is why the flight can be agile.

- Optimal goal assignment for swarms (Kushleyev et al., 2013) — deciding which robot takes which formation slot by minimizing total squared travel. This is the cooperation.

### Why small robots are agile

The talk’s central physics argument deserves to be internalized now, because it explains a design choice baked into quadsim/params.py: the simulated vehicle is a 33-gram Crazyflie-class machine, not a camera drone. Scale a quadrotor’s linear dimension by r. Mass goes as volume, m∼r3. Moment of inertia is mass times length squared, J∼r5. Rotor thrust scales roughly with the vehicle’s cube, and torque is thrust times arm length, so τ∼r4. Angular acceleration is therefore

ω=τJ∼r4r5=1r.

Halve the robot and it can reorient roughly twice as fast. Agility is not a software feature; it is a consequence of being small. Every gram matters — which is why Module 6 will care intensely about a 6-gram printed frame.

### How this course works

The method, agreed in docs/LEARNING.md, is read, run, break, explain. Each chapter you read a little theory, run real code, deliberately sabotage something to see the failure mode, and then explain what you saw in your own words. The “break” step is not a gimmick: seeing a simulation flip over because of one sign error teaches more, and costs less, than the same mistake on a real maiden flight. Homework goes into your Word copy of each chapter — every question has an answer box. Never move to the next chapter while the current one feels like magic.

### Why SPEC.md exists

One file deserves special explanation. SPEC.md is the project’s binding interface contract: it fixes names, function signatures, units, frames, and sign conventions before any module was written, so that independently authored modules plug together. Its own words:

Every module MUST follow this spec exactly: same names, signatures, units, frames, conventions. Multiple authors write modules concurrently against this contract.

You have met this idea before: it is an ICD, a bus specification, a datasheet. In robotics the stakes are concrete — if controller.py thinks the quaternion is body→world and dynamics.py thinks it is world→body, nothing crashes, no exception is raised, the vehicle simply flies into the floor. Conventions are load-bearing. That is also why this course takes its notation table (below) from SPEC.md rather than from any textbook: the course must match the code.

## Guided code walkthrough: a tour of the repository

Open README.md first; its layout section is the map:

quadsim/            core library
  maths.py          SO(3)/quaternion utilities (hat, vee, quat ops, rotation log)
  params.py         Crazyflie-2-like physical parameters
  dynamics.py       rigid-body dynamics, motor mixing/saturation, RK4 step
  trajectory.py     minimum-snap piecewise polynomials, flat outputs, yaw modes
  controller.py     geometric SE(3) tracking controller
  sim.py            simulation loop and History logging/metrics
  swarm.py          formation keyframes, optimal assignment, collision avoidance
  viz.py            3D trajectory plots, tracking panels, GIF animation
demos/              four runnable demos (write into out/)
tests/              pytest suite (maths, dynamics, trajectory, controller, swarm)
docs/HARDWARE.md    sim-to-real roadmap (Crazyflie levels 0-3)
SPEC.md             the binding interface contract the modules are written against

Notice the library has a strict layering (SPEC.md, “Global conventions”): maths and params sit at the bottom; dynamics builds on them; controller builds on maths/params/trajectory; sim orchestrates dynamics + controller + trajectory; swarm sits on everything; viz alone may touch matplotlib. Each chapter of this course climbs one layer of that stack — the course outline is the import graph.

Now trace how a picture gets made, because you will run demos constantly. Every demo has the same skeleton; here is the heart of demos/demo_hover.py:

params = QuadParams()
controller = SE3Controller(params)

target = np.array([0.0, 0.0, 1.0])
# Initial offset with ||offset|| = 0.4 m, split over all three axes.
offset = 0.4 * np.array([1.0, -1.0, 1.0]) / np.sqrt(3.0)
state0 = QuadState.hover(target + offset)
ref_fn = hover_ref(target)

T = 3.0 if args.fast else 6.0
hist = simulate(params, controller, ref_fn, state0, T, dt=0.002)

Physical parameters, a controller, a reference to follow, an initial state — then one call to simulate. That function, in quadsim/sim.py, is the closed loop you drew in every controls course, discretized at 500 Hz:

for k in range(n):
    t = k * dt
    f_cmd, tau_cmd = controller.compute(state, ref_fn, t)
    f_act, tau_act = dyn.unmix(dyn.mix(f_cmd, tau_cmd))
    ...
    state = dyn.step(state, f_cmd, tau_cmd, dt)

Controller reads the state and the reference, outputs a wrench (f,τ); the dynamics saturate it through the motor mixer (unmix(mix(...)) — Chapter 2), integrate one step, and loop. Everything is recorded into a History object — arrays of t,p,v,q,ω,f,τ plus the reference — and the demo then hands that history to quadsim/viz.py functions like plot_tracking(hist, ...) and animate_quads([hist], ...), which write the PNGs and GIFs you see in out/. That is the whole answer to “where do the GIFs come from”: demo → simulate → History → viz → out/. Nothing in out/ is hand-made; delete the folder and any demo will regenerate its part.

Demos end by printing metrics as name: value lines and a verdict — PASS, or WARN with a reason if a target from SPEC.md was missed. The targets (hover steady-state RMS < 0.005 m, figure-eight tracking RMS < 0.10 m, swarm separation > 0.15 m…) are listed in the README’s demo table, so a demo is simultaneously a movie and a regression test.

## The notation contract (memorize this page)

All symbols below are lifted directly from SPEC.md and used by every later chapter. SI units throughout: m, s, kg, N, rad.

| Symbol | Meaning | Convention (binding) |
| --- | --- | --- |
| world frame | fixed inertial frame | z up; gravity is −ge3 with g=9.81 |
| e3 | third basis vector | np.array([0.0, 0.0, 1.0]) |
| p,v | position, velocity | expressed in the world frame, shape (3,) |
| R∈SO(3) | attitude rotation matrix | maps body → world: vworld=R vbody |
| q=[w,x,y,z] | attitude quaternion | Hamilton convention, unit norm, scalar first, body→world |
| b3=Re3 | body z-axis | the thrust axis of the vehicle |
| f≥0 | total thrust (scalar, N) | produces world-frame force f⋅b3 |
| τ | body torque, (3,), N·m | expressed in the body frame |
| ω (code: w) | angular velocity, (3,), rad/s | expressed in the body frame |
| m,J,L | mass, inertia matrix, arm length | QuadParams: 0.033 kg, diag inertia, 0.046 m |

Three of these rows cause essentially all real-world quadrotor bugs, so stare at them now: R is body→world (many texts use the transpose); q stores the scalar part first (scipy stores it last); and ω lives in the body frame (the gyro’s frame — a real IMU is bolted to the body, which is exactly why the convention exists).

## Experiments

Run everything from the project root, /Users/vishalbharti/Downloads/micro-agile-robot-2026, with the project interpreter .venv/bin/python.

Experiment 1 — prove the toolchain works.

cd /Users/vishalbharti/Downloads/micro-agile-robot-2026
.venv/bin/python -m pytest tests/ -q

Observe: 27 passed in about 10–25 s depending on your machine. This is your testbench; run it after any change you make in this course. If it passes, the physics, math, controller, trajectories, and swarm all still meet spec.

Experiment 2 — first flight.

.venv/bin/python demos/demo_hover.py --fast

Observe the three metric lines (settle_time_s, steady_rms_m, final_error_m), the two saved: paths, and PASS. Open out/hover_tracking.png: the top panel shows x,y,z converging to the dashed references — a textbook second-order step response, which is no coincidence (Chapter 3). Then run the other three demos with --fast to see the whole show: demo_minsnap.py, demo_figure8.py, demo_swarm.py.

Experiment 3 — sanity-check a number by hand.

.venv/bin/python -c "
from quadsim.params import QuadParams
p = QuadParams()
print('weight_N:', p.weight)
print('f_max_N:', p.f_max)
print('per_motor_hover_N:', p.weight / 4)
"

Observe: weight mg=0.033×9.81≈0.324 N, maximum total thrust 0.64 N. Their ratio, 0.64/0.324≈1.98, is the thrust-to-weight the README rounds to “~2”, and each motor idles near 0.081 N at hover — about 51% of its 0.16 N limit. That headroom is the agility budget.

Experiment 4 — break it: the wrong interpreter. Deliberately run a demo with your system Python instead of the venv:

python3 demos/demo_hover.py --fast

Observe: on this machine it dies with an ImportError from inside matplotlib (an Anaconda/numpy version clash); on other machines you may get ModuleNotFoundError: quadsim. Either way, nothing about the project is broken — only the toolchain choice. This is the number-one failure you will ever hit here, and the fix is always the same: use .venv/bin/python, where pip install -e ".[dev]" installed the package editable against pinned-compatible libraries. File a mental note of what the error looks like so you recognize it instantly.

Experiment 5 — break it: move the hover target. Edit demos/demo_hover.py line 55 and change the target height from 1.0 to 2.5:

target = np.array([0.0, 0.0, 2.5])

Re-run .venv/bin/python demos/demo_hover.py --fast and look at out/hover_3d.png. The robot still starts 0.4 m from the target (the offset is relative) and settles in essentially the same time — the controller does not care where in space the setpoint is, only about the error. Now change it back and re-run to confirm you restored it. Getting comfortable with edit → run → inspect → revert is the skill of this module.

## Homework

Write your answers in the Word copy of this chapter.

Q1. In your own words (no peeking), reproduce the scaling argument: why does halving a quadrotor’s size roughly double its angular acceleration? State which quantity scales as r3, r4, and r5.

Your answer:

Q2. From Experiment 3: at hover, each motor produces about 0.081 N. What happens to the vehicle’s ability to translate aggressively as battery weight is added, given f_motor_max = 0.16 N is fixed? Express your answer using thrust-to-weight ratio.

Your answer:

Q3. SPEC.md says R maps body→world. Suppose one module silently used the transpose (world→body) when computing the thrust direction b3=Re3 for a tilted vehicle. Would any Python error occur? What would you observe in out/hover_3d.png instead? Why is this class of bug worse than a crash?

Your answer:

Q4. Trace the artifact chain for demos/demo_figure8.py by reading the file: which function call produces out/figure8.gif, which produces out/figure8_tracking.png, and what single object (name the class) carries the data from the simulation into both?

Your answer:

Q5. Scipy’s scipy.spatial.transform.Rotation stores quaternions as [x,y,z,w] (scalar last), while this project uses [w,x,y,z] (scalar first). Write the one-line index shuffle that converts a quadsim quaternion into scipy’s order, and explain what a “successfully wrong” conversion would do to a rotation of, say, 90° about z.

Your answer:

Q6. The control loop in quadsim/sim.py runs at dt=0.002 s. Using your DSP instincts: name one thing that could go wrong if we raised dt to 0.05 s (20 Hz) while the vehicle flies the aggressive figure-eight. (You will test this for real in Chapter 3.)

Your answer:

## Hints

- Start from ω=τ/J and scale numerator and denominator separately.

- Thrust-to-weight is p.f_max / p.weight; recompute it with m′=m+Δm.

- At hover, R≈I, and I⊤=I — so when would the two conventions first disagree?

- grep -n "animate\|plot_\|simulate" demos/demo_figure8.py shows every relevant call; the carrier class is defined in quadsim/sim.py.

- Numpy fancy indexing: q[[1, 2, 3, 0]]. For the 90° case, ask which component the wrong order treats as the scalar cos(θ/2).

- Think sampling a fast-changing reference, and phase lag around a feedback loop — the figure-eight demands attitude changes with significant energy above a few hertz.

## Further reading

- V. Kumar, “Robots that fly … and cooperate,” TED 2012 (https://www.youtube.com/watch?v=4ErEBkj_3PY) — 16 minutes; watch it tonight. It is the design brief for this entire repository, and you now know where each demo appears in it.

- README.md “Theory primer” section — a four-paragraph preview of Chapters 2–5; reading it now plants the vocabulary before the math arrives.

- docs/HARDWARE.md — skim Level 0 and Level 1 only, to see where this course is headed once the simulator holds no more mysteries: these very trajectories on a real 33 g Crazyflie.

- Mellinger & Kumar, “Minimum Snap Trajectory Generation and Control for Quadrotors,” ICRA 2011 — do not read it yet; just look at the figures. By Chapter 4 you will implement-read the whole thing.

# Chapter 1 — Rotations: quaternions are phasors, one dimension up

## After this chapter you can …

- Build the unit quaternion for any axis-angle rotation and explain where the half-angle comes from.

- Compose rotations by quaternion multiplication, apply them to vectors via quat_to_rot, and predict when order matters.

- Demonstrate gimbal lock numerically — and explain why the simulator’s state carries a quaternion instead of three Euler angles.

- Read q=12 q⊗[0,ω] as the 3D version of the phasor derivative rule, and integrate it yourself.

- Use hat/vee to move between vectors and skew-symmetric matrices, and linearize a small rotation as R≈I+θ.

## The ECE bridge

Every idea in this chapter is one you already own, promoted one dimension.

| You know (ECE) | This chapter |
| --- | --- |
| A phasor ejθ is a unit complex number; multiplying phasors adds phases | A unit quaternion q represents a 3D rotation; multiplying quaternions composes rotations (quat_mul) |
| ddtejθ=(jω) ejθ — multiply by an imaginary number to rotate | q=12 q⊗[0,ω] — multiply by a pure quaternion to rotate (quat_derivative) |
| Scalar transfer functions commute: H1H2=H2H1; MIMO (matrix) systems do not | 2D rotations commute (they’re scalars in C); 3D rotations do not — they’re matrices |
| An oscillator needs amplitude control (AGC) or numerical drift grows | An integrated quaternion needs quat_normalize or ∥q∥ drifts off 1 |
| atan2(0,0): phase of a zero-amplitude signal is undefined — a coordinate failure, not a physical one | Gimbal lock: at 90° pitch the Euler chart degenerates — the attitude is fine, its coordinates aren’t |
| Writing convolution as a Toeplitz matrix: recasting an operation as a linear operator | hat(v) recasts the cross product v×​ as a matrix, so calculus and linear algebra apply |
| Small-signal linearization around a bias point | R≈I+θ for small θ — the small-angle model every attitude controller lives on |

## The theory

Start in 2D, where you are at home. A rotation of the plane by θ is multiplication by z=ejθ=cosθ+jsinθ. Two facts make phasors so pleasant: composing rotations is just multiplying, ejθ1ejθ2=ej(θ1+θ2), and the constraint “this is a pure rotation, no scaling” is simply |z|=1. The set of unit complex numbers is a circle; rotations live on that circle, and calculus on it is easy: ddtejθ=jω ejθ, a velocity tangent to the circle, always perpendicular to the current phasor.

3D rotations want the same treatment, but they are bigger. A rotation in 3D has three degrees of freedom (an axis, two numbers, plus an angle). The honest representation is a 3×3 rotation matrix R with R⊤R=I and detR=1 — nine numbers tied down by six constraints. Matrices compose correctly but are clumsy to integrate: numerically drift off the constraint and you get a matrix that shears your vehicle instead of rotating it.

Quaternions are the 3D unit complex numbers. A quaternion q=[w,x,y,z] has one real part and three imaginary parts (i,j,k) with i2=j2=k2=ijk=−1. Restrict to unit norm and you get exactly the phasor deal: a compact object where multiplication composes rotations. The rotation by angle θ about unit axis n is

q=[cosθ2, sinθ2 n].

Why the half angle? Because a quaternion rotates a vector by the sandwich product [0,v′]=q⊗[0,v]⊗q−1 — q acts twice, once from each side, so each copy carries half the angle. One curious consequence: q and −q produce the same physical rotation (the “double cover”); you will verify this in the homework.

Order matters. In 2D, rotations commute because complex multiplication does. In 3D they don’t: roll 20° then pitch 50° is a genuinely different attitude than pitch 50° then roll 20°. You have met this before — scalar transfer functions commute in cascade, but matrix (MIMO) systems don’t. 3D rotation is inherently a matrix phenomenon, and quaternion multiplication faithfully inherits the non-commutativity.

Why not Euler angles? Three angles (yaw, pitch, roll) are the minimal parametrization, and minimal parametrizations of curved spaces always have a singular point — just as atan2(0,0) is undefined even though the signal is perfectly fine. For the aerospace zyx convention the bad point is pitch =±90°: the yaw axis and the roll axis line up, one degree of freedom vanishes from the coordinates (not from the physics), and infinitely many angle triples describe one attitude. A quadrotor doing a flip passes through that point at speed. Quaternions have no singular point anywhere, which is why the simulator’s 13-dimensional state vector carries [p,v,q,ω], not Euler angles.

The derivative rule. If the body spins with angular velocity ω (expressed in the body frame), the attitude quaternion evolves as

q=12 q⊗[0,ω].

Read it next to ddtejθ=ejθ⋅(jω): multiply the current orientation by a purely imaginary object built from the angular rate. The 12 is the half-angle again, now in differential form. And just as jωejθ is tangent to the unit circle, q is orthogonal to q — the motion stays on the unit sphere in 4D exactly, in continuous time. Discrete integration steps off the sphere by a tiny amount each step, which is why dynamics.step renormalizes — the AGC of attitude integration.

hat, vee, and small angles. The map hat:R3→3×3 skew-symmetric matrices, v u=v×u, turns the cross product into a linear operator. Its payoff is the small-angle model: for a small rotation vector θ,

R=eθ≈I+θ,

the rotational sibling of ex≈1+x, and the small-signal linearization on which attitude control (Module 3) rests. vee is the inverse — it extracts the 3 independent numbers hiding in a skew-symmetric matrix. Keep that phrase in mind for the homework question about eR.

## Guided walkthrough: quadsim/maths.py

Open quadsim/maths.py — 130 lines, the whole rotation toolkit. The module docstring fixes the conventions; misreading any of these is the classic source of sign bugs:

"""Conventions (see SPEC): quaternions are ``[w, x, y, z]`` (Hamilton, unit norm, body->world);
rotation matrices map body to world; angular velocity is expressed in the body frame."""

hat is the cross product in a matrix costume — compare each row against the determinant formula for v×u you know:

def hat(v: np.ndarray) -> np.ndarray:
    """Return the (3,3) skew-symmetric matrix such that ``hat(v) @ u == np.cross(v, u)``."""
    v = np.asarray(v, dtype=float)
    return np.array([
        [0.0, -v[2], v[1]],
        [v[2], 0.0, -v[0]],
        [-v[1], v[0], 0.0],
    ])

quat_mul is the Hamilton product written out. The first line is a dot-product-like combination for the real part; the remaining three mix a scalar-times-vector part with a cross product — this cross product is precisely where non-commutativity enters:

return np.array([
        w1 * w2 - x1 * x2 - y1 * y2 - z1 * z2,
        w1 * x2 + x1 * w2 + y1 * z2 - z1 * y2,
        ...

quat_to_rot converts q to a matrix, and hides a detail you will need for Homework 1:

w, x, y, z = np.asarray(q, dtype=float)
    s = 2.0 / (w * w + x * x + y * y + z * z)

The textbook formula uses s=2 and assumes ∥q∥=1. Dividing by ∥q∥2 instead makes the conversion self-normalizing: feed it a slightly off-unit quaternion and it still returns a proper rotation matrix. Remember this.

quat_derivative is the phasor rule verbatim:

def quat_derivative(q: np.ndarray, w: np.ndarray) -> np.ndarray:
    """Return the (4,) quaternion time derivative ``0.5 * q * [0, wx, wy, wz]``."""
    w = np.asarray(w, dtype=float)
    return 0.5 * quat_mul(q, np.array([0.0, w[0], w[1], w[2]]))

Finally, rot_to_quat (Shepperd’s method: branch on the largest of trace and diagonal so the square root and division are always well-conditioned — a numerical-robustness idiom worth stealing) and rot_log, which returns the axis-angle vector θ n of a rotation matrix. rot_log is how the controller will later measure “how far apart are these two attitudes” as a single 3-vector.

Where does it plug in? quadsim/dynamics.py line 107 computes q_dot = quat_derivative(q, w) inside the state derivative, and step closes the loop after RK4 integration:

x_new = x + (dt / 6.0) * (k1 + 2.0 * k2 + 2.0 * k3 + k4)
        x_new[6:10] = quat_normalize(x_new[6:10])

## Experiments

These are the six live-session experiments (A–F). Run everything from the repo root, /Users/vishalbharti/Downloads/micro-agile-robot-2026. The outputs shown are real, captured on this machine.

Experiment 1 (A + B) — the half-angle. Warm up with phasors, then build quaternions:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.maths import quat_mul
np.set_printoptions(precision=4, suppress=True)
z30, z45 = np.exp(1j*np.deg2rad(30)), np.exp(1j*np.deg2rad(45))
print("e^(j30) * e^(j45) angle:", round(np.rad2deg(np.angle(z30*z45)), 4), "deg")
def qrot(axis, deg):
    axis = np.asarray(axis, float); axis /= np.linalg.norm(axis)
    th = np.deg2rad(deg)
    return np.concatenate([[np.cos(th/2)], np.sin(th/2)*axis])
print("q(z,30)           =", qrot([0,0,1], 30))
print("q(z,30) * q(z,45) =", quat_mul(qrot([0,0,1],30), qrot([0,0,1],45)))
print("q(z,75) directly  =", qrot([0,0,1], 75))
EOF

Observed output:

e^(j30) * e^(j45) angle: 75.0 deg
q(z,30)           = [0.9659 0.     0.     0.2588]
q(z,30) * q(z,45) = [0.7934 0.     0.     0.6088]
q(z,75) directly  = [0.7934 0.     0.     0.6088]

Multiplication added the angles — the phasor property survived the trip to 3D. Break it: in qrot, replace th/2 with th (full angle) and add from quadsim.maths import quat_to_rot; then quat_to_rot(qrot([0,0,1],45)) @ [1,0,0] returns [0. 1. 0.] — a 90° rotation from a “45°” quaternion. The sandwich product applied your angle twice. This is the single most common quaternion bug in the wild.

Experiment 2 (C) — composition works; order doesn’t commute.

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.maths import quat_mul, quat_to_rot, rot_log
np.set_printoptions(precision=4, suppress=True)
def qrot(axis, deg):
    axis = np.asarray(axis, float); axis /= np.linalg.norm(axis)
    th = np.deg2rad(deg)
    return np.concatenate([[np.cos(th/2)], np.sin(th/2)*axis])
q75 = quat_mul(qrot([0,0,1],30), qrot([0,0,1],45))
print("R(q75) @ [1,0,0] =", quat_to_rot(q75) @ np.array([1.0,0,0]))
qa, qb = qrot([1,0,0],20), qrot([0,1,0],50)
Rab, Rba = quat_to_rot(quat_mul(qa,qb)), quat_to_rot(quat_mul(qb,qa))
print("|R(qa qb) - R(qa)R(qb)|_max =", f"{np.abs(Rab - quat_to_rot(qa)@quat_to_rot(qb)).max():.2e}")
print("|R(qa qb) - R(qb qa)|_max   =", f"{np.abs(Rab - Rba).max():.3f}")
print("actual angle between the two orders:", round(np.rad2deg(np.linalg.norm(rot_log(Rab.T @ Rba))), 3), "deg")
EOF

Observed: R(q75) @ [1,0,0] = [0.2588 0.9659 0. ] (that is [cos75°,sin75°,0]), the composition check is 1.11e-16 (machine epsilon — quaternion multiplication and matrix multiplication are the same operation in different clothes), and the swapped order differs by 0.262 in matrix entries — 16.834 deg of genuine physical disagreement. Try it with a book on your desk: roll then pitch vs pitch then roll.

Experiment 3 (D) — gimbal lock, and a lesson in trusting numbers.

.venv/bin/python - <<'EOF'
import numpy as np
from scipy.spatial.transform import Rotation as Rot
r1 = Rot.from_euler("zyx", [10.0, 89.9, 0.0], degrees=True)
r2 = Rot.from_euler("zyx", [-80.0, 89.9, 90.0], degrees=True)
print("difference:", round(np.rad2deg((r1.inv() * r2).magnitude()), 4), "deg")
EOF

Observed: difference: 0.1414 deg. Two wildly different angle triples — yaw 10° vs −80°, roll 0° vs 90° — describe essentially the same physical attitude. At 90° pitch, only the combination yaw+roll survives (here 10+0=−80+90); the individual angles are meaningless. A confession from the live session: our first guess was that the invariant is yaw−roll, and the check with [-80, 89.9, -90] returned 179.9999 deg — not “roughly equal,” but exactly upside down. The number didn’t negotiate; the narrative had the sign wrong. Break it yourself: rerun with roll = -90 and watch the 180° appear. When a clean number contradicts your story, the number is doing you a favor.

Experiment 4 (E) — integrate the derivative rule.

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.maths import quat_derivative, quat_normalize, quat_to_rot
np.set_printoptions(precision=4, suppress=True)
q, w, dt = np.array([1.0,0,0,0]), np.array([0,0,np.deg2rad(90)]), 0.001
for _ in range(1000):
    q = quat_normalize(q + dt * quat_derivative(q, w))
print("after 1 s at 90 deg/s: R @ [1,0,0] =", quat_to_rot(q) @ np.array([1.0,0,0]))
EOF

Observed: [0. 1. 0.] — spin about z at 90°/s for one second and the body x-axis lands exactly on world y. Break it: replace quat_derivative(q, w) with quat_mul(q, np.array([0.0, *w])) (dropping the 0.5). The result becomes [-1. 0. 0.] — 180° instead of 90°. The half-angle factor is not a convention you can shrug off; it is load-bearing.

Experiment 5 (F) — hat, cross, and the small-angle model.

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.maths import hat, quat_to_rot
w, u = np.array([1.0,2,3]), np.array([0.5,-1,2.0])
print("hat(w) @ u =", hat(w) @ u, "  np.cross(w,u) =", np.cross(w,u))
th = np.deg2rad(0.1)
q = np.array([np.cos(th/2), 0, 0, np.sin(th/2)])
print("small-angle error:", f"{np.abs(quat_to_rot(q) - (np.eye(3)+hat([0,0,th]))).max():.2e}")
EOF

Observed: both products give [ 7. -0.5 -2. ], and R≈I+θ holds to 1.52e-06 at 0.1°. Raise the angle to 10° and then 90° to watch the linearization degrade — that decay curve is exactly why large-tilt maneuvers need the geometric controller of Module 3 rather than a small-angle PID.

Experiment 6 — the whole session in one go. The original live script is at /private/tmp/claude-501/-Users-vishalbharti-Downloads-micro-agile-robot-2026/7406182a-5a7e-4e6b-bf26-9dff77fdca73/scratchpad/module1_rotations.py (scratchpad copies are session-temporary, so the snippets above are the durable record). If it still exists, run it and diff its output against the numbers embedded here.

## Homework

1. In quadsim/dynamics.py, comment out the renormalization line in step (x_new[6:10] = quat_normalize(x_new[6:10]), line 124), run .venv/bin/python demos/demo_figure8.py --fast --no-gif, and record the RMS error. docs/LEARNING.md predicts the sim will “slowly explode.” Explain what you actually observe, reconcile it with that prediction, and state under what conditions the missing normalization would bite. Restore the line afterwards.

Your answer:

2. The attitude error in quadsim/controller.py line 120 is e_R = 0.5 * vee(R_cmd.T @ R - R.T @ R_cmd) — exactly 3 numbers, although a rotation matrix has 9 entries. Why is 3 the right count, and what role does vee play in getting there?

Your answer:

3. Show that q=12q⊗[0,ω] is always orthogonal to q (numerically is fine: sample random unit q and ω, check np.dot(q, quat_derivative(q, w))). What is the 2D phasor analogue of this fact, and what does it imply about ddt∥q∥ in continuous time?

Your answer:

4. Verify numerically that quat_to_rot(q) and quat_to_rot(-q) return the same matrix for several random unit quaternions. Why do q and −q encode the same rotation, and why does rot_to_quat in quadsim/maths.py force w≥0 before returning?

Your answer:

5. Predict, then test: repeat Experiment 3 at pitch −89.9° instead of +89.9°. Is the gimbal-lock invariant yaw+roll or yaw−roll there? Write your prediction down before running anything, and report whether the numbers agreed with you.

Your answer:

## Hints

- Re-read the s = 2.0 / (w*w + x*x + y*y + z*z) line in quat_to_rot, then ask: how fast does RK4 at dt=2 ms actually drift the norm, and in what arithmetic (float32? seconds vs hours?) would that answer change?

- Count degrees of freedom, not matrix entries; then check what kind of matrix Rcmd⊤R−R⊤Rcmd always is.

- [0,ω] has zero real part; compare with jω being purely imaginary, and recall what ddt∥q∥2=2 q⊤q tells you.

- Look at the sandwich q⊗[0,v]⊗q−1 and count how many sign flips cancel; the w≥0 choice is about returning one consistent representative of the pair.

- Flipping the pitch sign flips which way the yaw and roll axes align at the singularity — expect the invariant’s sign to flip too.

## Further reading

- Kuipers, Quaternions and Rotation Sequences — written for engineers and builds from complex numbers upward, the same road this chapter took.

- Solà, Quaternion kinematics for the error-state Kalman filter (arXiv:1711.02508) — the free reference PDF everyone keeps open; it will pay off again when Module 7 fuses IMU data.

- Ben Eater & 3Blue1Brown, Visualizing quaternions (eater.net/quaternions) — interactive; the fastest way to see the double cover from Homework 4.

- Lee, Leok, McClamroch, Geometric tracking control on SE(3) (CDC 2010), §2 only for now — the origin of e_R; you will read the rest in Module 3.

# Chapter 2 — Rigid-body dynamics: thirteen numbers and four knobs

## After this chapter you can …

- Write down the quadrotor’s 13-state model [p,v,q,ω] from memory and say what each block does physically.

- Rederive every row of the mixer matrix from τ=r×f for the X layout, and predict which way the vehicle tips when you push on any motor.

- Explain why ω×Jω appears in Euler’s equation and when it actually matters for this vehicle.

- Compute the hover thrust per motor, the peak angular acceleration, and the thrust-to-weight ratio straight from params.py.

- Make Kumar’s “small is agile” claim quantitative: state how inertia, torque, and angular acceleration scale with vehicle size.

## The ECE bridge

| New concept | What you already know |
| --- | --- |
| State [p,v,q,ω], dynamics x=f(x,u) | A nonlinear state-space system, exactly like capacitor voltages and inductor currents in a circuit — 13 states, 4 inputs |
| Quaternion kinematics q=12 q⊗[0,ω] | The phasor derivative ddtejθ=jω ejθ: the derivative of a rotation is the rotation times the rate |
| Mixer matrix A and its inverse | A change of basis between channels — like a differential/common-mode decomposition, or beam-space vs element-space in an antenna array. Four motor thrusts on one side, thrust + three torques on the other, connected by an invertible 4×4 linear map |
| Gyroscopic term ω×Jω | The cross-coupling terms ωLiq, −ωLid in field-oriented motor control: differentiating in a rotating frame always adds an ω× term. Same product rule, one dimension up |
| Actuator saturation np.clip | Op-amp rail clipping / PA compression: the command is linear until it isn’t, and everything interesting (and dangerous) happens at the rails |
| RK4 vs Euler integration | Discretizing a continuous filter: forward Euler is cheap and drifts; higher-order methods buy accuracy per sample, like bilinear vs forward-difference transforms |
| Small-is-agile scaling | Dennard scaling. Shrink a transistor and capacitance falls faster than drive current, so small transistors switch faster. Shrink a quadrotor and inertia falls faster than torque, so small quadrotors roll faster |

## Core theory

### Thirteen numbers

A rigid body free in space needs its position and orientation, plus their rates. The state here is x=[p,v,q,ω]: position p∈R3 and velocity v∈R3 in the world frame, attitude quaternion q∈R4 (body→world, Hamilton convention [w,x,y,z] from Chapter 1), and angular velocity ω∈R3 in the body frame. That is 3+3+4+3=13 numbers for a system with only 12 degrees of freedom — the quaternion carries one redundant number, paid for by the constraint ∥q∥=1. The simulator re-imposes that constraint by renormalizing after every step; Chapter 1’s “break it” showed what happens when you don’t.

The four knobs are not the motors directly but the wrench they jointly produce: total thrust f along the body z-axis and body torque τ=[τx,τy,τz]. Four knobs, six degrees of freedom: the vehicle is underactuated. It cannot accelerate sideways without first tilting its thrust axis — which is why attitude dynamics sit inside every position maneuver, and why Chapter 3’s controller is cascaded.

### Newton and Euler

Newton’s part is translation, done in the world frame where momentum is simplest:

p=v,  v=−g e3+fm R e3.

Gravity pulls straight down; thrust pushes along the body z-axis b3=Re3, wherever the attitude currently points it. Nothing else — this model has no aerodynamic drag, which is a deliberate simplification you should remember when we go to hardware.

Euler’s part is rotation. Angular momentum in the body frame is H=Jω with inertia matrix J. The torque law H=τ holds in the inertial frame, but J is only constant in the body frame, so we apply the rotating-frame product rule — the transport theorem: Hinertial=Hbody+ω×H. Rearranged:

Jω=τ−ω×Jω.

That ω×Jω is the gyroscopic term, and it is exactly the jω your phasors pick up, or the ωLi cross-coupling in dq-frame motor control: any vector described in a spinning frame appears to rotate even when nothing physical is torquing it. The classic picture is a spinning bicycle wheel held by its axle: it carries a large angular momentum vector along the axle, and when you yaw the axle, that momentum vector must swing — which requires a torque, so the wheel fights back by pitching. Rotation about one axis, while momentum exists about another, produces apparent torque about the third. One subtlety worth a mental note: this term is about the momentum of the whole rigid body. The spinning propellers carry their own angular momentum too, and this model neglects it — a real, if small, sim-to-real gap.

For our vehicle J=diag(1.43×10−5, 1.43×10−5, 2.89×10−5) kg m2: a flat X shape, so the z inertia is roughly the sum of the other two. For a diagonal J the gyroscopic term is (ωyωz(Jzz−Jyy), ωxωz(Jxx−Jzz), ωxωy(Jyy−Jxx)) — with Jxx=Jyy its z-component is identically zero, and the whole thing vanishes at hover where ω≈0. It matters when you yaw while rolling or pitching: aggressive, coupled maneuvers.

### The mixer, row by row

The motors produce four individual thrusts f1..f4; the physics wants [f,τ]. The mixer matrix A is the bookkeeping between them. In the X configuration, each arm of length L=46 mm points along a diagonal, 45° from the body axes, so each rotor sits at x,y offsets of ±d with d=Lcos45°=L/2≈32.5 mm. Numbering runs counterclockwise seen from above: rotor 1 front-left (+d,+d,0), rotor 2 rear-left (−d,+d,0), rotor 3 rear-right (−d,−d,0), rotor 4 front-right (+d,−d,0).

Each rotor pushes fi along body z, so its torque about the center of mass is τi=ri×fie3. Work the cross product for a rotor at (xi,yi,0):

(xi,yi,0)×(0,0,fi)=fi (yi, −xi, 0).

Read the rows straight off this:

- Row 1 (thrust): total thrust is just the sum, [1,1,1,1].

- Row 2 (τx, roll): coefficient yi per motor: rotors 1, 2 sit at y=+d, rotors 3, 4 at y=−d, giving [d,d,−d,−d]. Sanity check: push harder on the two left rotors and τx>0, which by the right-hand rule rotates +y toward +z — the left side lifts. Correct.

- Row 3 (τy, pitch): coefficient −xi: rotors 1, 4 at x=+d contribute −d, rotors 2, 3 at x=−d contribute +d, giving [−d,d,d,−d]. Push the two rear rotors, the nose dips forward — pitch.

- Row 4 (τz, yaw): this one is not a lever-arm effect. Each spinning rotor drags air around and receives an equal-and-opposite reaction torque about its own axis, proportional to its thrust: τdrag=cτfi, with cτ=6 mm playing the role of an effective moment arm. Rotors 1 and 3 spin CCW (reaction on the body: −z), rotors 2 and 4 spin CW (+z), giving [−c,c,−c,c]. Diagonal pairs share a spin direction so that at hover the four reactions cancel; yawing means deliberately unbalancing them.

Because A is invertible, the controller can think entirely in [f,τ] and let mix solve f1..4=A−1[f,τ] — the same move as designing in beam-space and converting to element feeds at the last moment.

### Saturation, once per step

Real motors cannot pull (fi≥0) and cannot exceed fmax=0.16 N each. The simulator clips each motor thrust to [0,0.16] and then maps the clipped motors back through A to get the wrench the vehicle actually receives. Crucially, this happens once per 2 ms step, before integration, and the result is held constant across the step — a zero-order hold, exactly like a DAC output between samples. That models reality (the motor command is updated at the control rate, not continuously) and it keeps the ODE right-hand side smooth over the step, which RK4’s accuracy guarantees quietly assume. Clipping inside the derivative would pretend the electronics re-decide mid-step and would feed RK4 a kinked function.

Note what clipping does to the torque: np.clip acts per motor, so when a demanded wrench hits the rails, the achieved [f,τ] is not a scaled-down version of the demand — it is distorted. Saturation doesn’t just weaken your authority, it bends it. You will watch this happen in Experiment 2.

### RK4 versus Euler

Forward Euler advances xk+1=xk+dt f(xk): one derivative evaluation, local error O(dt2). Classic Runge–Kutta 4 blends four evaluations — at the start, two midpoints, and the end — for local error O(dt5). With dt=2 ms and angular accelerations that reach hundreds of rad/s2, Euler’s drift would masquerade as vehicle behavior; RK4 makes integration error negligible so that everything you observe is physics or control, not numerics. Four derivative calls per step is the price, and for 13 states it is nothing.

### Small is agile, quantitatively

Now Kumar’s scaling argument, with our numbers. Scale every length of the vehicle by s, same materials. Mass scales with volume: m∝s3. Inertia is mass times length squared: J∝s5 — check against params.py: mL2=0.033×0.0462=7.0×10−5, and Jxx=1.43×10−5≈0.2 mL2, a sensible geometric factor. Rotor thrust scales with disk area times tip speed squared; at constant tip speed (Mach scaling) F∝s2, so torque, thrust times lever arm, scales as τ∝s3. Therefore peak angular acceleration scales as

αmax=τmaxJ∝s3s5=1s2.

(Under Froude scaling, vtip∝s, you get 1/s — either way, shrinking wins.) For our vehicle: max roll torque is two motors at full thrust against two at zero, τxmax=2dfmotor max=2×0.0325×0.16≈0.0104 N·m, so αmax=0.0104/1.43×10−5≈730 rad/s2 — about 42,000°/s2, a 90° bank from rest in roughly 66 ms. Scale this design up 10× (0.46 m arms, ~33 kg) and αmax collapses by 100× to ~7 rad/s2. This is why the agile-flight literature is full of palm-sized vehicles, and why this project simulates a 33-gram one.

## Guided code walkthrough

Open quadsim/dynamics.py. The state is a plain dataclass with vector round-tripping (as_vector / from_vector) so the integrator can treat it as one flat R13 array. The equations of motion are almost a transliteration of the theory:

# quadsim/dynamics.py — derivative()
v_dot = -self.params.g * _E3 + (f / self.params.m) * (R @ _E3)
q_dot = quat_derivative(q, w)
w_dot = self._J_inv @ (tau - np.cross(w, self._J @ w))

Line 1 is Newton, line 3 is Euler with the gyroscopic term written exactly as J−1(τ−ω×Jω); quat_derivative is Chapter 1’s 12q⊗[0,ω]. The mixer is precomputed in the constructor:

# quadsim/dynamics.py — __init__()
d = params.L / np.sqrt(2.0)
c = params.c_tau
self.A = np.array([
    [1.0, 1.0, 1.0, 1.0],
    [d, d, -d, -d],
    [-d, d, d, -d],
    [-c, c, -c, c],
])

Compare each row against the derivation above; they must match sign for sign. mix applies A−1 then clips per motor; unmix maps motors back to the wrench. The step glues it together:

# quadsim/dynamics.py — step()
f_act, tau_act = self.unmix(self.mix(f, tau))

That single line is the actuator model: demand in, achievable wrench out, held constant while the four RK4 stages run, then quat_normalize repairs the unit constraint.

quadsim/params.py holds the Crazyflie-2-like numbers: m = 0.033, L = 0.046, c_tau = 0.006, f_motor_max = 0.16, plus two conveniences, f_max (=4fmotor max=0.64 N) and weight (=mg≈0.324 N). Divide those: thrust-to-weight ≈1.98. Hold that number; Experiment 2 spends it.

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026. The sed -i '' syntax is macOS; always run the revert line, and verify with the final grep.

Experiment 1 — hover arithmetic. Ask the mixer for pure hover thrust and no torque:

.venv/bin/python -c "
import numpy as np
from quadsim.params import QuadParams
from quadsim.dynamics import QuadrotorDynamics
p = QuadParams()
dyn = QuadrotorDynamics(p)
motors = dyn.mix(p.m * p.g, np.zeros(3))
print('per-motor thrust [N]:', motors)
print('m*g/4 =', p.m * p.g / 4)
print('fraction of f_motor_max:', motors[0] / p.f_motor_max)
"

Observe: all four motors at exactly mg/4=0.0809 N — the mixer’s first row summed against equal motors — sitting at ~51% of the 0.16 N rail. Half the throttle range is head-room for maneuvering. That margin is the next experiment’s victim.

Experiment 2 — the mass ladder (break it: doubled mass). The demo’s controller reads the same QuadParams, so it always knows the mass; what changes is what the motors can deliver.

.venv/bin/python demos/demo_minsnap.py --fast --no-gif    # baseline m=0.033: rms ~0.0006 m PASS
sed -i '' 's/m: float = 0.033/m: float = 0.0165/' quadsim/params.py
.venv/bin/python demos/demo_minsnap.py --fast --no-gif    # half mass: rms ~0.003 m PASS
sed -i '' 's/m: float = 0.0165/m: float = 0.040/' quadsim/params.py
.venv/bin/python demos/demo_minsnap.py --fast --no-gif    # +21%: rms ~0.10 m WARN
sed -i '' 's/m: float = 0.040/m: float = 0.066/' quadsim/params.py
.venv/bin/python demos/demo_minsnap.py --fast --no-gif    # doubled: rms ~8.8 m, gone
sed -i '' 's/m: float = 0.066/m: float = 0.033/' quadsim/params.py   # ALWAYS restore
grep 'm: float' quadsim/params.py                          # must print 0.033

Observe the cliff: halving the mass barely changes anything, +21% already breaches the 0.08 m target, and +50% or more is a crash. Why so nonlinear? At m=0.066, hover alone needs 0.162 N per motor against a 0.16 N rail — thrust-to-weight 0.99, the vehicle cannot even hold altitude. At m=0.040 hover is fine but the course’s peak accelerations push motors into the rails, and per-motor clipping distorts the commanded torque. This is the sabotage: it teaches that saturation is a cliff, not a slope, and that Kumar’s “small is agile” has a hard flip side — grams are a budget.

Experiment 3 — the mixer sign flip (break it: maiden-flight crash, in sim). Flip motor 1’s sign in the roll row, exactly the miswiring or mis-numbered-motor error that destroys real vehicles on their first takeoff:

sed -i '' 's/\[d, d, -d, -d\]/[-d, d, -d, -d]/' quadsim/dynamics.py
.venv/bin/python demos/demo_hover.py --fast --no-gif
sed -i '' 's/\[-d, d, -d, -d\]/[d, d, -d, -d]/' quadsim/dynamics.py
grep -n 'd, d, -d, -d' quadsim/dynamics.py                 # must show the restored row

Observe: baseline hover settles in ~1.2 s; sabotaged, settle_time_s: nan and a final error near 28 m — the vehicle flips and departs. The controller asks for a small corrective roll, the corrupted mixer delivers partly the opposite roll, the error grows, the “correction” grows: positive feedback, exactly an inverted feedback polarity in an op-amp loop. One sign, total loss. This is why real builds get a hand-on-the-vehicle prop-direction check before the first flight.

## Homework

- The state has 13 numbers but the vehicle has 12 degrees of freedom. Where is the redundancy, what constraint removes it, and which exact line of quadsim/dynamics.py enforces that constraint?

Your answer:

- Rederive the pitch row [−d,d,d,−d] of A from τi=ri×fie3, showing the cross product for rotor 1 explicitly. Then explain in one sentence, without math, why pushing harder on the front rotors gives negative τy.

Your answer:

- For our J with Jxx=Jyy, show that the z-component of ω×Jω is always zero, and describe one flight maneuver where the remaining components are large. What would change if the propellers’ own spin momentum were modeled too?

Your answer:

- Compute this vehicle’s maximum yaw angular acceleration from params.py the way the chapter computed the roll value (~730 rad/s²). Roughly how many times weaker is yaw, and what is the physical reason?

Your answer:

- Suppose step clipped motor thrusts inside derivative at every RK4 stage instead of once before integrating. Name two distinct things that would be wrong with that, one about physics and one about numerics.

Your answer:

- Hands-on: predict, then verify. If you doubled only f_motor_max to 0.32 N (mass unchanged), what happens to hover thrust per motor, thrust-to-weight, and the Experiment 2 crash at m=0.066? Make the edit, run demo_minsnap.py --fast --no-gif at m=0.066, record the result, and restore both values.

Your answer:

## Hints

- Count constraints, not just numbers — and look at the last two lines of step.

- The pitch coefficient is −xi; rotor 1 sits at x=+d.

- Write out ω×Jω for diagonal J and watch (Jyy−Jxx) appear; think “yaw while rolling” for the maneuver.

- Max yaw torque is 2 cτfmotor max; compare cτ=6 mm with d=32.5 mm, and remember yaw uses Jzz.

- ZOH is a claim about the electronics; RK4’s error bound is a claim about smoothness of the right-hand side.

- Hover per motor doesn’t depend on the rail at all; the crash was a thrust-budget problem, so predict which symptom disappears.

## Further reading

- Kumar & Michael, “Opportunities and challenges with autonomous micro aerial vehicles,” IJRR 2012 — the small-is-agile scaling laws from the source, including the Froude/Mach distinction this chapter compressed.

- Mahony, Kumar & Corke, “Multirotor aerial vehicles: Modeling, estimation, and control of quadrotor,” IEEE Robotics & Automation Magazine 2012 — the standard tutorial for exactly the model dynamics.py implements, plus the aerodynamic effects it leaves out.

- Mellinger & Kumar, “Minimum snap trajectory generation and control for quadrotors,” ICRA 2011, §II — the same dynamics written in the notation Chapters 3–4 will use; read the model section now, the rest later.

- Press et al., Numerical Recipes, ch. 17 (ODE integration) — a practical, engineer-friendly account of why RK4 earns its four function evaluations.

# Chapter 3 — Control: from the PID you know to geometry you’ll love

You already know feedback control; you learned it with op-amps and transfer functions. This chapter shows you that quadsim/controller.py is the same discipline — pole placement, cascaded loops, feedforward — with one genuinely new idea on top: doing PD control directly on the rotation group so the math survives aggressive tilt. Read with the file open beside you.

## After this chapter you can …

- Convert the controller’s kp, kv, kR, kw gains into natural frequencies and damping ratios, and predict step-response overshoot before running the sim.

- Explain the cascade — position loop commands attitude, attitude loop commands torque — and compute the ~8× time-scale separation from the actual default gains.

- Walk through every term of the Lee-2010 SE(3) control law as implemented in quadsim/controller.py, including the thrust projection and both feedforward terms.

- Say precisely why Euler-angle control fails at large tilt and what the matrix error eR fixes.

- Predict what actuator saturation does to a feedback loop, and demonstrate it.

## The ECE bridge

| New concept | What you already know |
| --- | --- |
| kp,kv position gains | Coefficients of a second-order characteristic polynomial s2+2ζωns+ωn2 — a series RLC step response with ωn=1/LC, ζ=R2C/L |
| Cascaded position/attitude loops | Inner current loop + outer voltage loop in an SMPS, or a PLL inside a data link: the inner loop must be fast enough to look like a unity-gain block to the outer one |
| Attitude error eR=12vee(Rc⊤R−R⊤Rc) | A phase detector: output ∝sin(Δϕ), linear for small error, well defined for any error |
| Feedforward ωff,αff | Pre-emphasis / feedforward compensation: inject the known part of the command so feedback only handles the unknown part |
| Finite-difference feedforward (rot_log(...)/d) | A discrete differentiator — ¦H(ejω)¦ grows with frequency, so it amplifies any kink or noise in the reference |
| Motor saturation in dynamics.mix | Amplifier clipping inside a feedback loop: loop gain collapses to zero the instant you clip, and the controller’s polite linear analysis is suspended |

## Core theory

### PD control is pole placement on a double integrator

Suppose the attitude loop were perfect, so the vehicle could produce any world-frame force instantly. Newton then gives mp=F−mge3, and if the controller chooses F=−kpep−kvev+mge3+mpref with ep=p−pref, the error obeys

mep+kvep+kpep=0 ⇔ s2+kvms+kpm=0.

Matching s2+2ζωns+ωn2: ωn=kp/m and ζ=kv/(2kpm). The defaults in controller.py are kp=16m, kv=8m, so

ωn=16=4 rad/s,  ζ=82⋅4=1.0.

Critically damped, bandwidth about 0.64 Hz. The gains are scaled by mass on purpose: the error dynamics — and every plot you make — stay identical if you change m in params.py. This is the same trick as normalizing a filter design to ωn=1.

### The cascade and the 8× rule

The vehicle cannot produce an arbitrary force: all four rotors point along body z, so it can only produce f Re3 — a magnitude along whatever direction it is currently tilted. Torque, however, it can produce in any body direction, and the body is light: torque changes tilt very fast. So we split the problem. The position loop computes the force it wishes it had, Fdes, and converts it into a commanded attitude Rcmd (tilt so body z points along Fdes). The attitude loop computes the torque that drives R→Rcmd.

The outer loop’s design pretended the inner loop was instantaneous. That lie is acceptable only if the inner loop is much faster. In compute, torque is τ=J(−kReR−kωeω)+…, and since Jω≈τ, the J cancels: the rotational error obeys e+kωe+kRe≈0 with the gains acting directly as ωn2 and 2ζωn (that is why kR, kw are not mass-scaled — the inertia is already factored out). With the defaults kR = [1000, 1000, 100], kw = [63, 63, 20]:

- roll/pitch: ωn=1000≈31.6 rad/s, ζ=63/(21000)≈1.0;

- yaw: ωn=100=10 rad/s, ζ=20/(2⋅10)=1.0.

Roll/pitch is what the position loop leans on, and 31.6/4≈7.9: the inner loop is 8× faster than the outer one. Within the outer loop’s 4 rad/s band, the attitude loop contributes almost no phase lag — exactly the assumption an op-amp circuit makes about the op-amp inside its gain-bandwidth. Erode that ratio (Experiment 1) and the outer loop starts eating its own phase margin. Yaw can afford to be slower because position tracking never depends on it.

### Why Euler angles break, and what SE(3) control does instead

The classic approach parameterizes attitude with roll-pitch-yaw and runs three little PID loops. The catch: the map from Euler-angle rates to body rates has a Jacobian containing 1/cos(pitch) terms — at 90° pitch it is singular (gimbal lock, Module 1), and well before 90° it is badly conditioned, so the “gains” the vehicle actually feels vary wildly with tilt. Euler PID is a linearization around hover; a quadrotor pulling through a fast figure-8 banks far outside that neighborhood.

The Lee–Leok–McClamroch controller never parameterizes. It keeps the attitude as a rotation matrix R∈SO(3) and defines the error on the group:

eR=12 vee​Rcmd⊤R−R⊤Rcmd.

Rcmd⊤R is the relative rotation “actual as seen from commanded”. Its antisymmetric part, extracted by vee, is a 3-vector of length sinθ per axis — your phase detector: linear near zero, smooth and well defined at any tilt, no singular Jacobian anywhere. PD on eR therefore behaves the same whether the vehicle is level or 60° banked. That is the whole pitch of geometric control: same PD instincts, error defined where the state actually lives.

### Saturation

The controller returns an idealized wrench (f,τ). Reality intrudes in quadsim/dynamics.py: mix solves for the four per-motor thrusts and clips each to [0,0.16] N, and unmix recomputes the wrench actually applied — which is what sim.py records in History.f and History.tau. Two consequences. First, thrust and torque share the same four motors, so a large roll-torque demand steals headroom from thrust and vice versa — the clipped wrench is not just “smaller”, it is distorted. Second, while any motor is clipped, that channel’s loop gain is effectively zero: the controller keeps shouting and nothing more happens, exactly like an op-amp stuck at its rail. Every divergence you will create in the experiments below passes through saturation on its way down.

## Guided code walkthrough — quadsim/controller.py

The spec for this file is SPEC.md § quadsim/controller.py; the code follows it line for line. Top to bottom:

flat_to_rotation(F, yaw) builds Rcmd from a desired world force and a heading:

b3 = F / norm_F
b1c = np.array([np.cos(yaw), np.sin(yaw), 0.0])
c = np.cross(b3, b1c)
...
b2 = c / np.linalg.norm(c)
b1 = np.cross(b2, b3)
return np.column_stack([b1, b2, b3])

Body z goes along the force; body x gets as close to the heading as the tilt allows. The two guards handle F≈0 (free fall — point up) and b3 parallel to the heading (pick a perpendicular heading instead), the two degenerate cross products.

Default gains in SE3Controller.__init__ are the numbers we analyzed:

self.kp = m * np.array([16.0, 16.0, 16.0]) if kp is None else ...
self.kv = m * np.array([8.0, 8.0, 8.0]) if kv is None else ...
self.kR = np.array([1000.0, 1000.0, 100.0]) if kR is None else ...
self.kw = np.array([63.0, 63.0, 20.0]) if kw is None else ...

compute — position loop. PD plus gravity plus acceleration feedforward, then the thrust projection:

F_des = -self.kp * e_p - self.kv * e_v + m * g * e3 + m * ref.acc
f = max(0.0, float(F_des @ (R @ e3)))
R_cmd = flat_to_rotation(F_des, ref.yaw)

The projection f=Fdes⋅Re3 is a subtle, important choice. The vehicle can only push along its current body z, which is Re3. Commanding ∥Fdes∥ would apply the full magnitude in the wrong direction while the attitude is still catching up; projecting commands exactly the useful component and lets the attitude loop rotate the thrust axis into place. The max(0, ...) is because rotors cannot pull.

Attitude feedforward. The commanded rotation is a pure function of time (feedback terms deliberately excluded so the reference stays deterministic), and its rate and acceleration are obtained by finite differences on the rotation group:

def R_ff(s: float) -> np.ndarray:
    r = ref_fn(s)
    return flat_to_rotation(m * r.acc + m * g * e3, r.yaw)

w_ff = rot_log(R_ff_0.T @ R_ff(t + d)) / d
w_ff_m = rot_log(R_ff(t - d).T @ R_ff_0) / d
a_ff = (w_ff - w_ff_m) / d

rot_log (Module 1) turns a small relative rotation into an axis-angle vector; dividing by d gives an angular rate, and differencing two one-sided rates gives an angular acceleration. This is the SO(3) version of x≈(x(t+d)−x(t))/d — with the same weakness: a differentiator amplifies reference roughness, which Experiment 3 will make visible.

Attitude loop. The two errors and the torque:

e_R = 0.5 * vee(R_cmd.T @ R - R.T @ R_cmd)
e_w = w - R.T @ R_cmd @ w_ff
tau = (
    J @ (-self.kR * e_R - self.kw * e_w)
    + np.cross(w, J @ w)
    - J @ (hat(w) @ R.T @ R_cmd @ w_ff - R.T @ R_cmd @ a_ff)
)

eω compares body rates in the same frame: ωff lives in the commanded body frame, so it is transported into the actual body frame by R⊤Rcmd before subtracting — you cannot subtract vectors expressed in different frames, any more than you can subtract phasors referenced to different carriers. In τ: the first term is the PD law (with J factored in so kR, kw are pure s−2, s−1 numbers); np.cross(w, J @ w) cancels the gyroscopic term ω×Jω from Euler’s equation (Module 2) so the loop sees a clean double integrator; the last line is the feedforward torque — the transported-rate term ωR⊤Rcmdωff accounts for the transport frame itself rotating (differentiating R⊤Rcmdωff by the product rule), and R⊤Rcmdαff injects the torque the maneuver is known to require, so feedback only cleans up the residual. The docstring’s last line is the saturation contract: “Actuator saturation is applied by the dynamics, not here.”

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026.

### Experiment 1 — starve the inner loop (break it: kR/16)

Scale kR down and watch the time-scale separation erode on a one-lap figure-8:

MPLBACKEND=Agg .venv/bin/python - <<'EOF'
import numpy as np
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.params import QuadParams
from quadsim.sim import simulate
from quadsim.trajectory import MinSnapTrajectory

th = np.linspace(0.0, 2.0 * np.pi, 17)          # one figure-8 lap, 16 waypoints
wp = np.column_stack([1.2 * np.sin(th), 1.2 * np.sin(th) * np.cos(th), np.full(17, 1.0)])
traj = MinSnapTrajectory(wp, avg_speed=2.5, yaw_mode="velocity")
params = QuadParams()
for name, scale in [("stock", 1.0), ("kR/2", 0.5), ("kR/4", 0.25), ("kR/16", 1 / 16)]:
    ctrl = SE3Controller(params, kR=np.array([1000.0, 1000.0, 100.0]) * scale)
    state0 = QuadState.hover(wp[0], yaw=float(traj(0.0).yaw))
    hist = simulate(params, ctrl, traj, state0, traj.T + 0.5)
    print(f"{name:6s} rms {hist.rms_pos_error():8.4f} m   max {hist.pos_error().max():8.3f} m")
EOF

Observe: RMS error goes 0.064 → 0.083 → 0.170 → 61 m. Halving kR drops the inner ωn to 500≈22 rad/s (separation 5.6×): the vehicle leans late into every turn and tracking degrades 30%. At kR/4 the ratio is ~4× and the path visibly weaves. The sabotage is kR/16: inner ωn=7.9 rad/s, barely 2× the outer loop — the cascade assumption collapses, the loops fight, saturation takes over, and the vehicle departs by 150 m. The 8× rule is not a convention; it is a stability margin.

### Experiment 2 — double kp with kv fixed (underdamped ringing)

Predict first: doubling kp gives ωn=32=5.66 rad/s and ζ=8/(2⋅5.66)=0.707 — the Butterworth point, ideal overshoot e−πζ/1−ζ2≈4%. Quadrupling gives ζ=0.5, ideal 16%. Now measure a 0.4 m step:

MPLBACKEND=Agg .venv/bin/python - <<'EOF'
import numpy as np
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.params import QuadParams
from quadsim.sim import simulate
from quadsim.trajectory import hover_ref
from quadsim.viz import plot_tracking

params = QuadParams()
for name, scale in [("stock", 1.0), ("kp_x2", 2.0), ("kp_x4", 4.0)]:
    ctrl = SE3Controller(params, kp=params.m * 16.0 * scale * np.ones(3))
    state0 = QuadState.hover(np.array([0.4, 0.0, 1.0]))     # 0.4 m step in x
    hist = simulate(params, ctrl, hover_ref(np.array([0.0, 0.0, 1.0])), state0, 4.0)
    overshoot = -hist.p[:, 0].min() / 0.4 * 100.0
    print(f"{name}: overshoot {overshoot:5.1f} %   final err {hist.pos_error()[-1]*1e3:.2f} mm")
    plot_tracking(hist, save=f"out/step_{name}.png", title=f"0.4 m step, {name}")
EOF

Observe: stock overshoots 0% (critically damped, as designed); kp_x2 overshoots 7.5%; kp_x4 overshoots 38% and rings — open the three out/step_*.png and compare the x traces to an RLC step response. What the deviation teaches: measured overshoot exceeds the ideal second-order prediction (7.5% vs 4%, 38% vs 16%) because the model assumed a perfect inner loop. Raising the outer ωn toward the attitude loop’s band makes its phase lag felt — you are watching time-scale separation erode from the outer side this time.

### Experiment 3 — zero the feedforward (break it: fly it fast)

Every use of rot_log inside compute builds feedforward, so monkeypatching it to zero kills w_ff and a_ff and nothing else (equivalently: edit controller.py, set them to zero, and revert after):

MPLBACKEND=Agg .venv/bin/python - <<'EOF'
import numpy as np
import quadsim.controller as C
from quadsim.dynamics import QuadState
from quadsim.params import QuadParams
from quadsim.sim import simulate
from quadsim.trajectory import MinSnapTrajectory

def lemniscate(laps, n):
    th = np.linspace(0.0, 2.0 * np.pi * laps, laps * n + 1)
    return np.column_stack([1.2 * np.sin(th), 1.2 * np.sin(th) * np.cos(th),
                            np.full(th.shape, 1.0)])

params = QuadParams()
real_rot_log = C.rot_log                          # keep a handle on the real one

def run(traj, wp, use_ff):
    C.rot_log = real_rot_log if use_ff else (lambda R: np.zeros(3))  # kill w_ff, a_ff
    ctrl = C.SE3Controller(params)
    state0 = QuadState.hover(wp[0], yaw=float(traj(0.0).yaw))
    hist = simulate(params, ctrl, traj, state0, traj.T + 0.5)
    pk = float(np.max(np.linalg.norm(hist.v, axis=1)))
    print(f"  {'with ff':8s}" if use_ff else f"  {'no ff':8s}",
          f"rms {hist.rms_pos_error():8.4f} m   peak speed {pk:6.2f} m/s")

print("gentle (demo-style: 16 wp/lap, velocity yaw)")
wp = lemniscate(1, 16)
traj = MinSnapTrajectory(wp, avg_speed=2.5, yaw_mode="velocity")
run(traj, wp, True); run(traj, wp, False)

print("aggressive (8 wp/lap, 0.5 s segments, fixed yaw)")
wp = lemniscate(2, 8)
traj = MinSnapTrajectory(wp, segment_times=np.full(wp.shape[0] - 1, 0.5))
run(traj, wp, True); run(traj, wp, False)
C.rot_log = real_rot_log
EOF

Observe — and be honest about it. On the gentle demo-style lap, removing feedforward improves RMS (0.064 → 0.031 m). Surprised? Good. At 2.5 m/s the tilts are small enough that feedback alone tracks fine, while the finite-difference αff is a double differentiator chewing on a reference whose velocity-mode yaw is only piecewise-smooth — it injects torque spikes (peaks over 250 rad/s² if you probe it). Feedforward is only as good as the smoothness of what you differentiate. The sabotage is the aggressive run: at 3.8 m/s peak, with feedforward the tracking is 0.043 m RMS; without it the vehicle diverges (RMS 87 m — a tumble through saturation). Feedback corrects errors after they appear; at aggressive speeds the errors appear faster than a 4 rad/s loop can answer, and the lean-into-the-turn-early torque must be supplied in advance or not at all.

## Homework

- Derive ωn and ζ for the vertical (z) position channel from the default gains. Why do the same numbers come out for x, y, and z even though gravity only acts on z?

Your answer:

- The attitude gains kR = [1000, 1000, 100] are not multiplied by mass or inertia in __init__ — yet the torque line starts with J @ (...). Explain why this makes the attitude error dynamics independent of J, and what units kR and kw therefore carry.

Your answer:

- Why does compute project thrust as F_des @ (R @ e3) instead of using np.linalg.norm(F_des)? Describe a concrete situation (vehicle level, Fdes 45° off vertical) and compare what each choice would command.

Your answer:

- In e_w = w - R.T @ R_cmd @ w_ff, what goes wrong physically if you write w - w_ff instead? Try it in a copy of the code on the aggressive figure-8 from Experiment 3 and report the RMS error.

Your answer:

- Experiment 2’s overshoot exceeded the ideal second-order prediction, and the gap grew with kp. Using the cascade story, explain the mechanism, and estimate at what outer ωn (in rad/s) you would expect the design to become unusable if the inner loop sits at 31.6 rad/s.

Your answer:

- In Experiment 3 the gentle run got better without feedforward. List the two independent reasons discussed in this chapter, and propose one change to the trajectory (not the controller) that would shrink the harmful one.

Your answer:

## Hints

- Write the closed-loop ODE for ez; note mge3 in F_des meets −mge3 in the dynamics before the error equation forms.

- Substitute τ into Jω=τ−ω×Jω and watch what cancels; then unit-check ωn2.

- Compute ∥Fdes∥cos45° vs ∥Fdes∥ applied along world z while R=I; which one overshoots in altitude?

- Frames. ω is expressed in the actual body frame; which frame is ωff expressed in?

- The inner loop contributes roughly arctan(ω/ωn,inner)-ish phase lag inside the outer crossover; think phase margin, and recall the 5–10× rule of thumb.

- One reason is about the differentiator, one about how hard feedback has to work; the trajectory knob is in how yaw is generated (see yaw_mode in Experiment 3’s two runs).

## Further reading

- Lee, Leok, McClamroch, Geometric Tracking Control of a Quadrotor UAV on SE(3), CDC 2010 — the six equations of compute with stability proofs; short and readable after this chapter.

- Mellinger & Kumar, Minimum Snap Trajectory Generation and Control for Quadrotors, ICRA 2011 — where Fdes-from-flat-outputs comes from; also Chapter 4’s main text.

- Åström & Murray, Feedback Systems, ch. 11 (PID) — the cleanest treatment of why derivative action is damping and how cascaded loops are tuned; free online.

- Crazyflie firmware controller_mellinger.c — this chapter’s controller in C on real hardware; diff it against controller.py as a preview of Module 7.

# Chapter 4 — Minimum snap: why the motion looks graceful

Chapter 3 gave you a controller that can chase a reference. This chapter is about making references worth chasing. The difference between a quadrotor that lurches between waypoints and one that flows through them like a swallow is not the controller — it is the trajectory. The key ideas are differential flatness (a structural gift of the quadrotor’s dynamics) and minimum-snap polynomial optimization (Mellinger & Kumar, ICRA 2011), and both live in quadsim/trajectory.py, our walkthrough file.

## After this chapter you can …

- State what differential flatness means for a quadrotor and trace the chain from position derivatives to attitude, body rates, and motor commands — and therefore explain why the fourth derivative (snap) is the thing to minimize.

- Count the degree-of-freedom budget of a piecewise 7th-order spline — unknowns vs constraints per segment — and say where the optimizer’s leftover freedom lives.

- Read the KKT linear system in _solve_coefficients as equality-constrained least squares and explain the role of the time scaling τ=(t−ti)/Ti.

- Retell the project’s real integration incident — the figure-8 whose yaw-rate demand exceeded vehicle authority — and replay its mechanism in simulation on today’s code.

- Design your own waypoint course, find the vehicle’s empirical speed limit, and verify a trajectory’s derivatives numerically.

## The ECE bridge

| New thing here | What you already know |
| --- | --- |
| Differential flatness: 4 outputs (x,y,z,ψ) determine all 13 states | Observability: for an observable system you can reconstruct the full internal state from the output and enough of its derivatives. Flatness is the constructive version — an algebraic formula, no observer needed |
| Minimizing ∫∥snap∥2dt | Differentiation multiplies a spectrum by jω; four derivatives means ω8 in the cost. Minimizing snap is a smoothness regularizer that crushes high-frequency content, like penalizing stopband energy in filter design |
| Equality-constrained QP via the KKT system | The MVDR beamformer: minimize wHRw subject to wHa=1, solved with Lagrange multipliers. Identical structure — quadratic cost, linear equality constraints, one bordered linear solve. Also: normal equations with constraints |
| Piecewise polynomials with derivative continuity at knots | Continuous-phase FSK: phase is kept continuous at symbol boundaries so the spectrum stays compact. Here derivatives 1–4 are kept continuous at waypoints so the actuator spectrum stays compact |
| Time scaling τ=(t−ti)/Ti∈[0,1] | Normalized frequency in filter design: design the prototype on [0,1], then frequency-scale. Same trick, same reason — numerical conditioning |
| Velocity-aligned yaw via atan2, np.unwrap, hold below 0.15 m/s | Instantaneous phase of an analytic signal, with unwrapping; the low-speed hold is a PLL holding its last phase estimate when the carrier drops below usable SNR |

## Core theory

### Differential flatness: four numbers rule them all

The quadrotor has 13 states and 4 inputs, and yet the planner in this project only ever thinks about four scalar functions of time: x(t),y(t),z(t),ψ(t) — position and yaw. That is legal because the quadrotor is differentially flat: every state and every input can be written as an algebraic function of these four flat outputs and finitely many of their time derivatives. No differential equation needs to be integrated to plan; planning becomes curve design.

The construction is worth internalizing because it is the controller’s feedforward path. Newton’s law for the translational dynamics reads mp=−mge3+fRe3, so the required world-frame force is

F=m (p+ge3),

and since the thrust can only point along the body z-axis, F hands us both the thrust magnitude f=∥F∥ and the direction b3=F/∥F∥. The yaw angle ψ then pins down the remaining rotation about b3, giving the full attitude R. So: acceleration determines attitude. Differentiate once more: the jerk p tells you how fast b3 is turning, i.e. the body angular rates ω. Differentiate again: the snap p determines the angular acceleration ω, which through τ=Jω+ω×Jω determines the torques, which through the mixer matrix of Chapter 2 determine the four individual motor thrusts.

That chain — position → velocity → acceleration → attitude → (jerk) body rates → (snap) motor commands — is the whole argument for minimum snap. Snap is the derivative that lands directly on the motors. Minimize ∫∥p∥2dt and you are, quite literally, minimizing actuator violence. This is why the flight looks graceful: the plan never asks the motors for anything abrupt.

### Why 7th-order polynomials

Apply calculus of variations to min∫(p)2dt and the Euler–Lagrange equation is p(8)(t)=0: the optimal curve between boundary conditions is a polynomial of degree 7. (The general pattern: minimizing the k-th derivative squared gives degree 2k−1; minimum jerk would give quintics.) So each spline segment is a 7th-order polynomial per axis — 8 coefficients, _N_COEFF = 8 in the code.

### The constraint budget

Take n segments (from M=n+1 waypoints). Per axis you have 8n unknown coefficients. The constraints imposed in quadsim/trajectory.py are:

- each segment starts and ends at its waypoints: 2n constraints;

- rest at both trajectory endpoints — velocity, acceleration, jerk all zero: 6 constraints;

- continuity of physical-time derivatives 1 through 4 at each interior knot: 4(n−1).

Total: 6n+2. The slack is 8n−(6n+2)=2(n−1) free parameters per axis — two per interior waypoint. Those are what the optimizer spends: it gets to choose the velocity, acceleration, jerk, and snap values at interior knots (subject to continuity) so as to minimize total snap energy. For n=1 the budget is exact (8=8): a single segment is fully determined by its boundary conditions and there is nothing to optimize.

### Constrained least squares and the KKT system

The cost is quadratic in the coefficients, cTHc, and all constraints are linear, Ac=b. You have solved this shape of problem before — MVDR beamforming, equality-constrained least squares — and the solution is the same here: introduce multipliers λ, set the gradient of the Lagrangian to zero, and solve one bordered linear system,

2HATA0cλ=0b.

This is the KKT system, and it is solved once per axis (the three axes conveniently share the same matrix, so the code solves for a 3-column right-hand side).

### Time scaling: the conditioning trick

If you wrote each segment’s polynomial in raw time t, a 16-second trajectory would need monomials up to t7≈2.7×108 next to constants of order one — a numerically disastrous Vandermonde-style spread. Instead each segment uses scaled time τ=(t−ti)/Ti∈[0,1], so every basis value stays O(1). The bookkeeping cost is the chain rule: a k-th physical-time derivative picks up Ti−k, and the snap cost picks up Ti−7 per segment. This is exactly your filter-design habit of working in normalized frequency and scaling afterwards.

That Ti−7 is worth staring at: halving a segment’s duration multiplies its snap cost by 128. Segment times matter enormously, which brings us to the incident.

### Segment times, the 0.5 s floor, and the figure-8 incident

The constructor allocates default segment times as Ti=max(di/v, 0.5) — segment length over requested average speed, floored at half a second. The floor exists because this vehicle (33 g, four motors of 0.16 N each, about 2:1 thrust-to-weight) simply cannot execute an arbitrarily short segment; Ti−7 says the demanded aggression explodes as Ti shrinks.

Here is the real story from this project’s integration, recorded in the git log (commit 3a06026: “retuned the figure-8 waypoint density (8 → 16 per lap) to keep yaw-rate demand within vehicle authority”). The original figure-8 demo sampled the lemniscate at 8 waypoints per lap. Every segment hit the 0.5 s floor, so two laps compressed into 8 s, with peak speeds near 3.9 m/s — and yaw_mode="velocity" obliged the nose to sweep the whole heading profile in half the time. The integration run clocked the demanded yaw rate around 22 rad/s — more than three revolutions per second, far beyond what the attitude loop can deliver while simultaneously banking hard. (Re-measuring on today’s code with the 500 Hz yaw grid you will see reference peaks in the mid-teens of rad/s; either number is well beyond authority.) The sim did not politely WARN; it diverged, RMS error ≈ 69 m. (That 69 m is the historical record from the integration run; the code has been tuned since, and replaying the 8-point case today — Experiment 4 — gives a degraded-but-stable RMS ≈ 0.25 m rather than outright divergence. The mechanism, though, is unchanged.) The fix was counterintuitive and instructive: densify the waypoints. At 16 points per lap there are twice as many floor-limited segments, so the same geometry takes 16 s instead of 8 s, peak speed drops to about 2.5 m/s, and the demo passes at RMS 46 mm. The docstring of demos/demo_figure8.py now records the outcome: dense sampling plus the 0.5 s floor put the whole-lap average near 0.9 m/s. Moral: with floor-dominated timing, waypoint density is the speed knob.

### Yaw modes

Two modes exist. "fixed" holds a constant heading — right for formation flying. "velocity" points the nose along the direction of travel, ψ=atan2(vy,vx) — the instantaneous phase of the horizontal velocity, treated exactly like a communications phase: computed on a dense 500 Hz grid, unwrapped with np.unwrap, differentiated with np.gradient to get the yaw rate, and held wherever horizontal speed drops below 0.15 m/s, because the phase of a near-zero vector is noise — a PLL coasting through a signal dropout.

## Guided code walkthrough: quadsim/trajectory.py

The planner’s public face is the FlatOutput dataclass — one flat-output sample:

pos: np.ndarray   # (3,)
vel: np.ndarray = field(default_factory=lambda: np.zeros(3))   # (3,)
acc: np.ndarray = field(default_factory=lambda: np.zeros(3))   # (3,)
jerk: np.ndarray = field(default_factory=lambda: np.zeros(3))  # (3,)
snap: np.ndarray = field(default_factory=lambda: np.zeros(3))  # (3,)
yaw: float = 0.0
yaw_rate: float = 0.0

Everything a controller needs, nothing it doesn’t. The polynomial machinery rests on one small function, _basis_row, which builds the row vector whose dot product with the 8 coefficients gives the k-th τ-derivative:

def _basis_row(k: int, tau: float) -> np.ndarray:
    """Row ``r`` with ``r @ c`` = k-th tau-derivative of ``sum_j c_j tau**j`` (8 coeffs)."""
    row = np.zeros(_N_COEFF)
    for j in range(k, _N_COEFF):
        row[j] = _falling(j, k) * tau ** (j - k)
    return row

Inside _solve_coefficients, the snap cost for one segment is a closed-form 8×8 Gram matrix — the integral 01τj τk dτ done analytically — and the Ti−7 scaling appears exactly where the theory said it would:

for j in range(4, _N_COEFF):
    for k in range(4, _N_COEFF):
        snap_block[j, k] = _falling(j, 4) * _falling(k, 4) / (j + k - 7)
...
    hess[sl, sl] = snap_block * t_i ** (-7)

The constraint rows follow the budget we counted. Note how the interior continuity rows equate physical-time derivatives by scaling each side with its own segment’s T−k — segment i’s end must match segment i+1’s start:

for i in range(n_seg - 1):
    for k in (1, 2, 3, 4):
        row = np.zeros(n_c)
        row[i * _N_COEFF:(i + 1) * _N_COEFF] = (
            _basis_row(k, 1.0) * self._times[i] ** (-k))
        row[(i + 1) * _N_COEFF:(i + 2) * _N_COEFF] = (
            -_basis_row(k, 0.0) * self._times[i + 1] ** (-k))

The KKT assembly is textbook — compare it symbol-for-symbol with the block matrix above:

kkt[:n_c, :n_c] = 2.0 * hess
kkt[:n_c, n_c:] = a_mat.T
kkt[n_c:, :n_c] = a_mat

Evaluation applies the chain rule one last time — every derivative order gets its Ti−k:

return [(_basis_row(k, tau) @ coeffs) * t_i ** (-k) for k in range(5)]

Finally, see how the flatness chain is consumed in quadsim/controller.py: the reference acceleration becomes force, and force plus yaw becomes the commanded attitude —

F_des = -self.kp * e_p - self.kv * e_v + m * g * e3 + m * ref.acc
f = max(0.0, float(F_des @ (R @ e3)))
R_cmd = flat_to_rotation(F_des, ref.yaw)

— which is F=m(p+ge3) wearing feedback goggles. The class is callable (__call__ = eval), so a MinSnapTrajectory instance drops straight into simulate as a ref_fn.

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026.

Experiment 1 — design your own gate course. First fly the stock course:

.venv/bin/python demos/demo_minsnap.py --no-gif

Note rms_error_m (~0.001), total_time_s, peak_speed_mps (~4.1), and look at out/minsnap_3d.png. Now open demos/demo_minsnap.py and edit the WAYPOINTS array: add a waypoint, move the finish, make the dive after gate 2 deeper — but keep W1 and W3 pinned to the gate centers [1.2, 0.0, 1.0] and [3.2, 0.8, 1.5] so you still thread the openings. Re-run the same command and check the 3D plot: does your path still clear the box? Notice the smooth curve never touches your straight polyline except at the waypoints — the optimizer bows the path to spread out curvature. When done, restore with git checkout -- demos/demo_minsnap.py.

Experiment 2 — raise avg_speed until it breaks. Find this vehicle’s limit empirically without editing files:

.venv/bin/python - <<'EOF'
import numpy as np, sys
sys.path.insert(0, '.')
from demos.demo_minsnap import WAYPOINTS
from quadsim.trajectory import MinSnapTrajectory
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.params import QuadParams
from quadsim.sim import simulate

for spd in (2.0, 2.2, 2.4, 2.6, 3.0, 6.0):
    traj = MinSnapTrajectory(WAYPOINTS, avg_speed=spd)
    params = QuadParams()
    hist = simulate(params, SE3Controller(params), traj,
                    QuadState.hover(WAYPOINTS[0]), traj.T + 1.0, dt=0.002)
    rms = float(hist.rms_pos_error())
    peak = float(np.max(np.linalg.norm(hist.v, axis=1)))
    print(f"avg_speed={spd:4.1f}  T={traj.T:5.2f}s  peak={peak:5.2f} m/s  rms={rms:.4f} m  "
          + ("WARN" if rms >= 0.08 else "ok"))
EOF

Observe: 2.2 m/s still tracks at 49 mm RMS, but 2.4 m/s is a cliff — RMS jumps to about 1.5 m (the vehicle saturates its motors and gives up, not gradually but catastrophically). Also observe that from 2.6 upward nothing changes: every segment has hit the 0.5 s floor (T=2.50 s), so avg_speed has become a dead knob. This experiment is the sabotage: you are deliberately commanding beyond vehicle authority and watching the failure mode — the same failure the figure-8 incident hit, found the same way.

Experiment 3 — trust, then verify, the derivatives. Evaluate a trajectory in a shell and finite-difference its position against its claimed velocity:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.trajectory import MinSnapTrajectory

wp = np.array([[0., 0., 1.], [1., 0., 1.], [1., 1., 1.5], [0., 1., 1.]])
traj = MinSnapTrajectory(wp, avg_speed=1.5)
h = 1e-5
worst = 0.0
for t in np.linspace(0.1, traj.T - 0.1, 200):
    fd_vel = (traj(t + h).pos - traj(t - h).pos) / (2 * h)
    worst = max(worst, float(np.linalg.norm(fd_vel - traj(t).vel)))
print("T =", traj.T, " max |fd_vel - vel| =", worst)
EOF

Expect agreement around 10−9. Break it: in quadsim/trajectory.py, in _derivatives, change * t_i ** (-k) to * 1.0 and re-run. The mismatch jumps to order one — you have deleted the chain rule, so the code returns τ-derivatives while physics needs t-derivatives. Also run .venv/bin/python demos/demo_minsnap.py --fast --no-gif in the broken state: the constructor’s fit check does not catch the bug (it only verifies position, the k=0 case, and Ti0=1), but the controller now receives feedforward velocity and acceleration in the wrong units of time and tracking collapses to a WARN at RMS ≈ 0.36 m — a silent-units bug, the classic kind. Restore with git checkout -- quadsim/trajectory.py.

Experiment 4 — replay the integration incident.

.venv/bin/python - <<'EOF'
import numpy as np, sys
sys.path.insert(0, '.')
from demos.demo_figure8 import lemniscate_waypoints
from quadsim.trajectory import MinSnapTrajectory
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.params import QuadParams
from quadsim.sim import simulate

for ppl in (8, 16):
    wp = lemniscate_waypoints(2, points_per_lap=ppl)
    traj = MinSnapTrajectory(wp, avg_speed=2.5, yaw_mode="velocity")
    params = QuadParams()
    hist = simulate(params, SE3Controller(params), traj,
                    QuadState.hover(wp[0], yaw=float(traj(0.0).yaw)),
                    traj.T + 0.5, dt=0.002)
    print(f"points_per_lap={ppl}: T={traj.T:.1f}s "
          f"ref_yaw_rate_peak={np.max(np.abs(traj._yaw_rates)):.1f} rad/s "
          f"rms={hist.rms_pos_error():.4f} m")
EOF

Observe: 8 points/lap gives T=8 s and RMS ≈ 0.25 m — more than double the demo’s 0.10 m WARN threshold; 16 points/lap gives T=16 s and RMS ≈ 0.046 m, centimeter-level tracking. Same lemniscate, same avg_speed argument; the only change is waypoint density interacting with the 0.5 s floor, which halves every segment time and pushes the peak speed from about 2.5 to 3.9 m/s. On today’s retuned code this replay shows a five-fold tracking degradation, not the full 69 m divergence of the historical incident. One caution when reading the printout: the single ref_yaw_rate_peak number is nearly identical in the two cases (≈ 16 vs ≈ 15 rad/s), because in both runs the raw peak comes from one brief low-speed moment (horizontal speed ≈ 0.4 m/s, above the 0.15 m/s hold threshold, so the yaw reference is not held) where the velocity heading swings fast. The sustained yaw-rate demand really is about double in the 8-point case (95th percentile ≈ 8.6 vs ≈ 4.1 rad/s), so judge the two runs by RMS and trajectory time, not by the peak alone.

## Homework

- For the six-waypoint course in demos/demo_minsnap.py (n=5 segments), compute the per-axis constraint budget: unknowns, constraints of each kind, and the optimizer’s leftover freedom. Where, physically, does that freedom live?

Your answer:

- Minimum snap uses 7th-order polynomials. Derive the polynomial order that a minimum-jerk planner would use, and explain via the Euler–Lagrange equation why minimizing the k-th derivative yields degree 2k−1.

Your answer:

- Why would minimum-velocity paths — straight lines between waypoints at constant speed — be terrible for a quadrotor? Answer using the flatness chain: what happens to attitude and body rates at each waypoint corner?

Your answer:

- The figure-8 docstring says dense sampling plus the 0.5 s floor put the whole-lap average near 0.9 m/s, even though avg_speed=2.5 is requested. Using the lemniscate geometry (16 chords/lap, a=1.2 m), estimate the lap length, confirm every segment hits the floor, and compute the actual average speed. Why is avg_speed a dead knob here?

Your answer:

- Hands-on: extend Experiment 3 to check acc against a finite difference of vel, sampling away from the interior knots and then straddling a knot. Derivatives 1–4 are continuous at knots by construction — which derivative is the first one allowed to jump, and can you detect the jump numerically?

Your answer:

- In _build_yaw_grid, yaw is held wherever horizontal speed is below 0.15 m/s. What goes wrong at the lemniscate’s center crossing if you remove the hold? Relate this to phase estimation of a weak carrier.

Your answer:

## Hints

- The counts are in the “constraint budget” section; the freedom is 2(n−1) — think about which quantities at interior knots the optimizer chooses rather than being told.

- min∫(p(k))2dt gives the Euler–Lagrange condition p(2k)=0; count the polynomial’s degrees of freedom from there.

- A corner means velocity changes direction instantaneously — what does the flatness chain say acceleration, hence attitude, must do, and what body rate does an instantaneous attitude change require?

- Chord length of one lap ≈ sum of 16 segment distances (roughly 7.2 m); each di/2.5<0.5 s, so Tlap=16×0.5=8 s.

- Continuity is enforced only through the 4th derivative, so the first quantity allowed to jump is the 5th; estimate it by finite-differencing snap across a knot.

- atan2 of a near-zero vector is noise, and at the crossing the velocity direction reverses; np.gradient of a step is a spike — of exactly the kind that broke the original figure-8.

## Further reading

- Mellinger & Kumar, Minimum snap trajectory generation and control for quadrotors, ICRA 2011 — the source paper; short and genuinely readable now that you’ve seen the code.

- Richter, Bry & Roy, Polynomial trajectory planning for aggressive quadrotor flight in dense indoor environments, ISRR 2013 — reformulates the QP in unconstrained form for better conditioning and optimizes segment times too; the natural “what’s next” after this chapter’s fixed-time solver.

- Boyd & Vandenberghe, Convex Optimization, §10.1 (equality-constrained minimization) — the KKT system in this chapter, stated with full generality in four pages.

# Chapter 5 — Swarms: nine robots, one dance

You have flown one quadrotor through a figure-eight and a gate course. This chapter flies nine at once, and the punchline is how little new control theory that requires. The swarm problem, as this repo frames it, decomposes into three ideas layered on top of everything you already built: a shared reference (the virtual leader), a matching problem (who takes which slot), and a safety reflex (short-range repulsion). Each quad still runs the same SE3Controller from Chapter 3, unmodified.

## After this chapter you can …

- Explain the virtual-leader + formation-offset architecture and why it turns a nine-robot coordination problem into nine independent tracking problems.

- Formulate slot assignment as bipartite matching, solve it with the Hungarian algorithm, and defend the choice of squared distances.

- Derive the quintic smoothstep and explain — pointing at a specific line of controller.py — why formation transitions must be C2 in time.

- Predict when the 0.30 m avoidance bubbles activate, and describe the standoff between the position loop and the repulsion term when they do.

- Say honestly which parts of this sim are decentralized in Kumar’s sense and which are centralized conveniences.

## The ECE bridge

Virtual leader + offsets is a phased array. In a beamforming array you never command each element independently; you command one phase center and each element holds a fixed geometric offset from it. Here the leader reference ℓ(t) is the phase center, and slot i is ℓ(t)+δi(t). Steering the array = flying the leader.

The assignment problem is channel allocation. Nine users, nine channels, a cost for every (user, channel) pair, each channel assigned to exactly one user, minimize total cost: that is resource allocation in OFDMA schedulers and crossbar-switch scheduling, and it is exactly minimum-cost bipartite matching. The Hungarian algorithm (via scipy.optimize.linear_sum_assignment) solves it in polynomial time — no need to enumerate 9!=362,880 permutations.

Smoothstep blending is pulse shaping. Keying a carrier with rectangular edges splatters energy across the spectrum, so you shape symbols with raised-cosine pulses whose derivatives vanish at the edges. A formation change is a keying event on the reference signal; the controller differentiates that signal (twice!), and differentiation multiplies spectra by (jω)n. Sharp edges in, torque splatter out.

Squared distances are least squares. Same reason as everywhere in estimation: the L2 cost is smooth, and it obeys the parallelogram law, which — you will see — is precisely what makes optimal assignments non-crossing.

Repulsion is Coulomb’s law with a gain: a 1/d potential-style push, applied reciprocally like a force pair, acting as a fast local loop underneath the slower global plan — the same layering as a current-limit loop inside a voltage regulator.

## Core theory

### Virtual leader and formation offsets

The central design decision in quadsim/swarm.py is that a formation is not nine trajectories. It is one trajectory — the leader flat output ℓ(t), a Callable[[float], FlatOutput] — plus a (9,3) array of offsets. Quad i’s reference is

piref(t)=ℓp(t)+δi(t),  viref(t)=ℓv(t)+δi(t),  airef(t)=ℓa(t)+δi(t).

Because references live in flat-output space (Chapter 4), they add by superposition: derivatives of a sum are sums of derivatives, so the leader’s motion and the formation’s shape change compose without any coupling. Each quad then tracks its own reference with its own controller and never needs to know the others exist — except through the repulsion term below. This is why the demo runs nine full SE3Controller + RK4 pipelines in lockstep and the code barely grows.

### The assignment problem, and why squared distances

When the formation changes from a grid to a ring, some robot must take some ring slot, but nothing says row-major order is a good pairing. A bad pairing sends robots across the formation through each other. So between consecutive keyframes we build the cost matrix

Cij=∥δiprev−δjnext∥2

and solve minπi​Ci π(i) over permutations π — minimum-cost perfect matching on a bipartite graph of robots and slots.

Why squared norms and not plain distances? Take any two robots at p1,p2 assigned to goals g1,g2. If that pairing is optimal under squared cost, swapping cannot help:

∥p1−g1∥2+∥p2−g2∥2≤∥p1−g2∥2+∥p2−g1∥2.

Expand both sides and the quadratic terms cancel, leaving

(p1−p2)⋅(g1−g2)≥0.

The displacement between the two robots and the displacement between their two goals never point in opposing directions — straight-line transitions don’t cross head-on. This is the heart of Turpin, Michael & Kumar’s CAPT result: squared-distance assignment plus synchronized straight-line motion guarantees separation, given enough initial clearance. A linear-distance cost gives no such inequality (the cross terms don’t cancel), and it also happily trades one very long trip for savings spread across others; the squared cost penalizes the longest trips hardest, which balances the transition. Cheap insurance, chosen for a theorem, not for taste.

### Smoothstep and the C2 requirement

Between keyframes, offsets blend as δ(t)=δprev+s(u) (δnext−δprev) with u=(t−tstart)/tblend and the quintic smoothstep

s(u)=6u5−15u4+10u3,  s′(0)=s″(0)=s′(1)=s″(1)=0.

It is the lowest-order polynomial meeting those six boundary conditions — five derivative constraints plus s(0)=0,s(1)=1 need six coefficients. Why insist on two vanishing derivatives? Look at quadsim/controller.py. The reference acceleration enters the force command directly (F_des = ... + m * ref.acc), so a jump in δ is a step in commanded thrust direction — ugly but survivable. The real killer is a few lines down:

d = fd_dt
R_ff_0 = R_ff(t)
w_ff = rot_log(R_ff_0.T @ R_ff(t + d)) / d
w_ff_m = rot_log(R_ff(t - d).T @ R_ff_0) / d
a_ff = (w_ff - w_ff_m) / d

The controller builds its attitude feedforward by finite-differencing the reference with d=10−3 s. A discontinuity in reference acceleration becomes a discontinuity in the commanded rotation Rff, and the second difference turns that into a spike of order 1/d2 in aff — a torque hammer. So the reference handed to this controller must be C2: position, velocity, and acceleration continuous. That is why _smoothstep returns all three of s,s′,s″ analytically, why the leader in the demo ramps its circle angle with the same quintic, and why SwarmSim refuses overlapping blend windows (two simultaneously active blends would need summing logic the single-blend loop doesn’t have — the guard fails loudly instead of flying wrongly). Experiment 4 puts a number on the damage a mere C0 blend does.

### Repulsion: the safety reflex

The plan above is open-loop about collisions: assignment and smooth blending make them unlikely, not impossible. So a last layer watches actual positions. For any pair closer than dsafe=0.30 m, each robot receives an acceleration along the separating direction,

apush=kavoid1dij−1dsafenij,

zero exactly at the bubble edge, growing without bound as dij→0, applied equal-and-opposite to both robots (reciprocal, like a force pair), and capped at 3 m/s2 per robot so it can never demand more than the actuators plausibly deliver. Crucially, it is injected as an extra acceleration on the reference, not as a raw force — it flows through the same controller, so the vehicle leans into the dodge properly. Note what it needs to compute: only the relative position of a nearby neighbor. That is a genuinely decentralizable rule.

### Honesty: how much of this is really “decentralized”?

Kumar’s talks celebrate decentralization; this sim keeps its flavor while quietly centralizing the hard parts. Genuinely local: the formation rule (each quad needs only the broadcast leader state and its own offset — like each array element needing only the reference oscillator) and the pairwise repulsion (relative positions of near neighbors, measurable onboard in principle). Centralized conveniences: the assignment is solved once, offline, at construction by an all-knowing planner; every quad reads exact ground-truth state (no estimation, no mocap noise, no latency); the simulation is lockstep-synchronous, i.e., perfect clock distribution and a lossless radio. And the bubble is a sphere — real multi-quad work uses ellipsoids stretched along z, because a quadrotor’s downwash hammers anyone below it; this demo dodges the issue by keeping every formation flat at one altitude. To be fair, the famous lab demos (Kushleyev et al. 2013) were also Vicon-tracked and centrally planned — only the kilohertz attitude loops ran onboard. The sim is honest company; just don’t mistake either for a GPS-denied forest swarm.

## Guided code walkthrough

Open quadsim/swarm.py. A formation change is data, not code:

@dataclass
class FormationKeyframe:
    t_start: float
    t_blend: float
    offsets: np.ndarray   # (n, 3), meters, relative to the leader

The assignment happens once, in SwarmSim.__init__, sequentially keyframe-to-keyframe — note prev is the already permuted previous formation, so identities chain correctly through the whole show:

prev = self._offsets[-1]
cost = ((prev[:, None, :] - cand[None, :, :]) ** 2).sum(axis=2)
_, col = linear_sum_assignment(cost)
self._offsets.append(cand[col])

Three lines: broadcasted squared-distance cost matrix, Hungarian solve, permute the candidate rows. blended_offsets(t) then walks the keyframes and returns offsets and their two analytic derivatives, which _formation_ref adds onto the leader’s flat output — including the avoidance term:

return FlatOutput(pos=base.pos + off[i], vel=base.vel + doff[i],
                  acc=base.acc + ddoff[i] + a_extra, ...)

a_extra is held constant in t deliberately: the controller will finite-difference this ref_fn, and a frozen avoidance term contributes zero to those differences — the reflex nudges the force loop without detonating the attitude feedforward. The repulsion itself is _avoidance_accels, a plain O(n2) pair loop implementing the formula above, with the per-robot norm cap at the end.

Now demos/demo_swarm.py. The three formations are small pure functions — grid_offsets() (3×3, 0.55 m spacing), ring_offsets() (radius 0.9 m, chord 2rsin(π/9)=0.62 m), and v_offsets(), a migrating-birds chevron whose docstring does the safety arithmetic for you (closest slots 0.495 m apart). make_leader builds the C2 leader: hover, then one circle lap whose angle is ramped by the same quintic smoothstep, so the lap starts and ends with zero velocity and acceleration, matching the hovers. The timeline is just three keyframes and a leader:

keyframes = [
    FormationKeyframe(t_start=0.0,  t_blend=1.0, offsets=grid_offsets()),
    FormationKeyframe(t_start=6.0,  t_blend=4.0, offsets=ring_offsets()),
    FormationKeyframe(t_start=12.0, t_blend=4.0, offsets=v_offsets()),
]
leader = make_leader(t0=18.0, t_circle=10.0)

Everything you watch in swarm.gif is those five lines plus the machinery above.

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026.

Experiment 1 — baseline. Establish the healthy numbers:

.venv/bin/python demos/demo_swarm.py --fast

(~16 s.) Observe: steady_formation_rms_m ≈ 0.0000, max_formation_error_m ≈ 0.0007, and min_pairwise_distance_m ≈ 0.4102 — comfortably above the 0.30 m bubble. The stock demo never triggers avoidance; every formation was designed with ≥ 0.4 m slot spacing on purpose. Look at out/swarm_3d.png.

Experiment 2 — your initials as a keyframe. Nine robots, nine dots: trace a letter. Template (a “V” — edit pts into your own letter, then try your other initial):

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.swarm import FormationKeyframe, SwarmSim, min_pairwise_distance, formation_errors
from demos.demo_swarm import grid_offsets, make_leader

def letter_V(scale=0.45):
    pts = [[0.0, -0.9, 0.0]]                                  # vertex at the bottom
    for k in range(1, 5):
        pts.append([-k * 0.5 * scale, -0.9 + k * scale, 0.0])  # left stroke
        pts.append([+k * 0.5 * scale, -0.9 + k * scale, 0.0])  # right stroke
    return np.array(pts)

off = letter_V()
d = np.linalg.norm(off[:, None, :] - off[None, :, :], axis=2)
d[d == 0] = np.inf
print("min slot spacing (m):", d.min())      # keep this >= 0.4!

keyframes = [
    FormationKeyframe(t_start=0.0, t_blend=1.0, offsets=grid_offsets()),
    FormationKeyframe(t_start=3.0, t_blend=3.0, offsets=letter_V()),
]
swarm = SwarmSim(QuadParams(), make_leader(t0=1e9, t_circle=10.0), keyframes)
hist = swarm.run(9.0, dt=0.004)
print("min_pairwise_distance_m:", min_pairwise_distance(hist))
print("max_formation_error_m:", float(formation_errors(hist).max()))
from quadsim.viz import plot_trajectory_3d
plot_trajectory_3d(hist, save="out/swarm_initials.png", title="Letter formation")
EOF

Observe: the spacing check first (0.45 m for this “V”), then min pairwise ≈ 0.446 m and max error under a millimeter. Print swarm._offsets[1] if you’re curious which grid robot the Hungarian algorithm sent to which stroke.

Experiment 3 — break it: shrink the ring into the bubbles. A 9-slot ring has chord spacing 2rsin(π/9)≈0.684 r; at r=0.35 that is 0.239 m < 0.30 m, so the requested formation itself violates the bubbles:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.swarm import FormationKeyframe, SwarmSim, min_pairwise_distance, formation_errors
from demos.demo_swarm import grid_offsets, ring_offsets, make_leader

keyframes = [
    FormationKeyframe(t_start=0.0, t_blend=1.0, offsets=grid_offsets()),
    FormationKeyframe(t_start=3.0, t_blend=3.0, offsets=ring_offsets(radius=0.35)),
]
swarm = SwarmSim(QuadParams(), make_leader(t0=1e9, t_circle=10.0), keyframes)
hist = swarm.run(10.0, dt=0.004)
errs = formation_errors(hist)
print("min_pairwise_distance_m:", min_pairwise_distance(hist))
print("max_formation_error_m:", float(errs.max()))
print("final error per quad (m):", np.round(errs[-1], 3))
EOF

Observe: min pairwise ≈ 0.213 m — the repulsion did not restore 0.30 m, but it kept robots off each other — and final formation errors of 3–10 cm that never settle. What it teaches: the safety layer and the position loop reach a standoff. The position loop pulls toward the slot with acceleration ≈(kp/m) ep=16 ep; the repulsion pushes back with kavoid(1/d−1/dsafe), capped at 3 m/s². Neither wins; the swarm hovers in a strained compromise, permanently off its formation. Safety layers degrade tracking by design — the fix is a better plan, not a bigger gain.

Experiment 4 — break it: replace the smoothstep with a linear ramp. Monkeypatch the blend to s(u)=u (a C0 blend: reference velocity jumps at the window edges):

.venv/bin/python - <<'EOF'
import quadsim.swarm as sw
sw._smoothstep = lambda u: (u, 1.0, 0.0)   # sabotage: linear, derivatives lie
from quadsim.params import QuadParams
from quadsim.swarm import FormationKeyframe, SwarmSim, formation_errors
from demos.demo_swarm import grid_offsets, ring_offsets, make_leader
keyframes = [
    FormationKeyframe(t_start=0.0, t_blend=1.0, offsets=grid_offsets()),
    FormationKeyframe(t_start=3.0, t_blend=3.0, offsets=ring_offsets()),
]
swarm = SwarmSim(QuadParams(), make_leader(t0=1e9, t_circle=10.0), keyframes)
hist = swarm.run(10.0, dt=0.004)
print("max_formation_error_m:", float(formation_errors(hist).max()))
EOF

Observe: max error jumps from 0.0007 m to ≈ 0.037 m — a 50× degradation — and the peak lands at t≈3.2 s, right at the blend’s leading edge, where the velocity step hits e_v and the finite-difference feedforward. Rectangular keying, torque splatter.

Experiment 5 — break it: overlap two blend windows. Start the second blend before the first finishes:

.venv/bin/python - <<'EOF'
from quadsim.params import QuadParams
from quadsim.swarm import FormationKeyframe, SwarmSim
from demos.demo_swarm import grid_offsets, ring_offsets, make_leader
keyframes = [
    FormationKeyframe(t_start=0.0, t_blend=2.0, offsets=grid_offsets()),
    FormationKeyframe(t_start=1.0, t_blend=3.0, offsets=ring_offsets()),
]
SwarmSim(QuadParams(), make_leader(t0=1e9, t_circle=10.0), keyframes)
EOF

Observe: ValueError: keyframe blend windows must not overlap ... at construction, before anything flies. What it teaches: blended_offsets handles exactly one active blend; overlapping windows would silently produce a reference that teleports between partial blends. Validating inputs at the boundary and failing loudly is a controls habit worth stealing.

## Homework

- Complete the proof from the theory section: expand ∥p1−g1∥2+∥p2−g2∥2≤∥p1−g2∥2+∥p2−g1∥2 and show it reduces to (p1−p2)⋅(g1−g2)≥0. Then give a 2-robot example where the linear-distance-optimal assignment makes their straight-line paths pass through the same point at the same time.

Your answer:

- In the fast demo (grid → ring), predict which grid robot takes which ring slot for at least the four corner robots, then check by printing swarm._offsets[1] next to grid_offsets(). Was the Hungarian solution the one you would have picked by eye?

Your answer:

- The blend’s peak offset speed is maxus′(u)⋅∥Δδ∥/tblend. Find maxus′(u) for the quintic analytically, then compute the fastest slot’s peak speed in the grid → ring transition of the fast demo (tblend=3 s).

Your answer:

- _formation_ref adds the avoidance acceleration a_extra held constant in t. Suppose instead ref_fn recomputed avoidance from live positions at each queried time. Explain, using the w_ff/a_ff lines of controller.py, why that seemingly “more correct” version is dangerous.

Your answer:

- The avoidance bubble is a 0.30 m sphere. Explain why a real multi-quadrotor system needs an ellipsoidal keep-out region instead, and identify the design choice in demo_swarm.py’s formations that lets the spherical bubble get away with it here.

Your answer:

- Hands-on: in Experiment 3, the equilibrium is a balance between the position loop’s pull and the repulsion’s push. Using kp/m=16 s−2, kavoid=4, dsafe=0.30, estimate the pairwise distance at which a robot with 0.05 m of formation error is in equilibrium with one neighbor, and compare with the measured final errors and distances.

Your answer:

## Hints

- The quadratic terms ∥pi∥2,∥gj∥2 appear on both sides. For the counterexample, try two robots and two goals arranged so both pairings have equal total linear distance — degenerate ties are where linear cost fails.

- Build the SwarmSim exactly as in the demo’s --fast branch, in an interactive shell. Row i of _offsets[1] is where grid robot i ends up.

- Set s″(u)=0: the extremum of s′ is at u=1/2, and s′(1/2) is a nice fraction. The fastest slot is the one with the largest ∥Δδ∥ after assignment — a corner robot.

- The finite differences evaluate ref_fn at t−d, t, and t+d with d=10−3. What does dividing a jump by d2 produce, and which command does a_ff feed?

- Think about what the propellers do to the air below the vehicle, then look at the z column of every offsets array in the demo.

- Set 16×0.05=4 (1/d−1/0.30) and solve for d. Remember each robot in a tight ring has two close neighbors whose pushes partially cancel tangentially.

## Further reading

- Kushleyev, Mellinger, Powers, Kumar — “Towards a swarm of agile micro quadrotors” (2013). The paper behind this module: formation keyframes, ellipsoidal (downwash-aware) separation, and what actually ran onboard vs. on the base station.

- Turpin, Michael, Kumar — “CAPT: Concurrent assignment and planning of trajectories” (IJRR 2014). The theorem that makes “why squared distances” a proof instead of a preference.

- Kuhn — “The Hungarian method for the assignment problem” (1955). Short, classic, and the reason linear_sum_assignment exists; skim to see the duality idea, which will feel like network-flow LP.

- Kumar’s TED talk, “Robots that fly … and cooperate” (2012). The nine-robot finale this demo is a nod to — watch it once before, and once after, this chapter.

# Chapter 6 — Mechanical design and CAD: the physical twin

Everything so far lived in numpy. This chapter is where the simulation grows a body: a 3D-printed frame, four brushed motors, and documents that promise the physical vehicle will behave like quadsim/params.py says it does. Mechanical design is genuinely new territory for you — so we lean hard on the fact that it runs exactly like an electronics project: one datasheet, parameterized source files, design-rule checks, and budgets.

## After this chapter you can …

- Trace any master dimension (33 g, L=46 mm, 92 mm diagonal, 65.05 mm adjacent spacing, T/W ≈ 2.0) from quadsim/params.py through hardware/frame.scad to out/frame_drawing.png, and explain why params.py is the single source of truth.

- Read and modify a parametric OpenSCAD model, re-render it, and export an STL — and say what its assert() lines protect you from.

- Explain the four big design-for-printing rules (support-free geometry, layer-adhesion anisotropy, press-fit tolerances, stiffness-per-gram) and point at the exact feature in frame.scad that implements each one.

- Read a dimensioned two-view engineering drawing: views, phantom lines, dimension chains, leaders, and the title block.

- Audit the weight and thrust budgets in docs/DESIGN.md, recompute the margins yourself, and catch the places where the hardware documents disagree with each other.

## The ECE bridge

| New thing here | What you already know |
| --- | --- |
| params.py as the master for all hardware dims | The datasheet / interface control document. Board layout never contradicts the ICD; here, CAD never contradicts params.py |
| Parametric CAD (OpenSCAD: geometry as code) | A parameterized Verilog module: change one parameter, every derived dimension recomputes. The frame.scad parameter block is a parameter block |
| Derived values + assert() in the SCAD file | localparam plus DRC/ERC: computed constants and design-rule checks that fail the build, not the flight |
| STL export vs the .scad source | Gerbers vs the schematic: STL is a dumb triangle mesh you send to the fab (printer); the design intent lives only in the source |
| Press-fit bore = 7.0 + 0.1 mm tolerance | PCB drill sizing: hole diameter = lead diameter + plating/fit allowance. Print shrinkage plays the role of plating |
| FDM layer anisotropy (weak between layers) | A laminate: strong in-plane, delaminates between plies — same reason PCB prepreg fails between layers, not through copper |
| Weight budget table (§6 of DESIGN.md) | A power budget or RF link budget: every entry measured, subtotaled, margin declared, and treated as a contract |
| Thrust-to-weight = 2.0 | Amplifier headroom / PA back-off: hover is the average signal, T/W is how far below saturation you operate |
| CW/CCW motor + prop pairing matched to the mixer | Polarity conventions: swapping one differential pair inverts the signal; swapping one prop hand inverts a torque column |

## Core theory

### One number to rule them: why every dimension traces to params.py

The controller of Chapters 3–5 was tuned against a specific plant: m=0.033 kg, L=0.046 m, J=diag(1.43,1.43,2.89)×10−5 kg·m², per-motor thrust capped at fmax=0.16 N. If the physical vehicle drifts from these numbers, the sim’s predictions — hover throttle, tracking error, the speed at which demo_minsnap starts to WARN — quietly stop applying. So the project adopts datasheet discipline: one file owns each number; everything else derives from it and is checked against it.

The derivations are short enough to own. The X frame puts the four rotors at angles 45°, 135°, 225°, 315°, each at radius L from the center. Opposite rotors are therefore 2L=92 mm apart (the diagonal), and each rotor sits at body coordinates (±d,±d) with d=L/2=32.53 mm — the very d in the allocation matrix of quadsim/dynamics.py, which uses d = params.L / np.sqrt(2.0) for its roll and pitch rows. Adjacent rotors are 2d=2L/2=65.05 mm apart. That last number decides the biggest prop you may fit: with no ducts, adjacent tips must never come closer than 10 mm, so prop diameter is capped at 65.05−10≈55 mm. And the thrust-to-weight ratio is pure params.py:

TW=4fmotor maxmg=4×0.160.033×9.81=1.98≈2.0

which is why every hardware document specs motors at ≥ 16.5 gf each — slightly above the sim’s 16.3 gf — so the mixer’s software clip, not the motor, is always the binding limit.

### Parametric CAD versus direct modeling

There are two ways to make a 3D model. Direct (history-based) modeling — Fusion 360, SolidWorks — is sculptural: you sketch, extrude, fillet, and the tool records a feature history. It is interactive and great for organic shapes, but the design intent (“this bore is motor diameter plus fit clearance”) lives in your head unless you carefully attach named parameters. Parametric code-CAD — OpenSCAD — is the opposite: the model is a program. Geometry is built from primitives (cube, cylinder) combined with union and difference, and every dimension is a named variable. Change arm_length, press render, and the entire frame — arms, ribs, motor rings, assert checks — recomputes. For someone who already thinks in parameterized HDL, code-CAD is the gentler entry, and it diffs cleanly in git, which is why frame.scad is the master geometry. The agreed follow-up (Module 6, LEARNING.md) is to later rebuild this frame in Fusion 360 with frame.scad as the dimension source — worth it when we want STEP files (the exchange format that preserves real surfaces, unlike STL’s triangle soup) or nicer renders. Same relationship as schematic → Gerber: you regenerate the export, you never edit it.

### Design for 3D printing

An FDM printer builds the part as a stack of ~0.2 mm molten plastic layers. Four consequences drive the whole frame design.

Overhangs. Each layer needs something under it. Walls leaning past roughly 45° from vertical droop and need sacrificial support material — ugly, weak, extra work. The frame sidesteps this completely: it has a flat base at z=0, every feature (arms, ribs, motor rings) rises straight up from the bed, and every chamfer or slot is cut from above. It prints support-free, flat side down.

Layer adhesion. The part is a laminate: strong along layers, weak between them (the inter-layer bond is maybe half the bulk strength). So you orient the part so that flight loads put layers in tension along their plane, never peeling them apart. Arms printed flat mean bending loads from thrust are carried by continuous in-plane filament roads.

Press fits. Printed holes come out 0.1–0.2 mm undersized (the perimeter squishes inward). The motor bore is therefore parametric — nominal can diameter plus an explicit tolerance — and there is a chamfered lead-in so the motor starts straight. If your printer runs tight, you change one number, not the geometry.

Stiffness per gram. The frame is ≈ 6 g of the 33 g budget — about 18 % — and it must be stiff, not strong: flexy arms couple motor vibration into the IMU and wreck attitude estimation. Bending stiffness of a thin section scales with thickness cubed, so the cheapest stiffness is a tall thin rib on top of each arm (a poor man’s I-beam), plus lightening holes where material carries no load. This is why the model spends parameters on rib_height and lightening_hole_diam rather than just making the arms fatter.

### The propulsion picture

The vehicle uses 7×16 mm brushed coreless motors (7 mm can, 16 mm long, ≈ 3.2 g each) driving ~55 mm press-fit props. At this scale brushed wins the system trade: the four drive FETs live on the flight controller board, so there are no separate ESCs; the motors cost ~$4 and are treated as consumables (brushes wear, thrust fades over hours). The price is a slower thrust response — a first-order lag of 30–60 ms the sim doesn’t model.

The part that must match the software exactly is handedness. In the mixer of quadsim/dynamics.py, rotors 1 and 3 (the +x+y / −x−y diagonal) spin CCW seen from above and contribute −c to the yaw row; rotors 2 and 4 spin CW and contribute +c. So the hardware needs 2 CCW + 2 CW motors and 2 CCW + 2 CW props, placed on the correct arms. Brushed FETs drive one polarity only, so spin direction is fixed by which motor variant you press into which ring — you cannot fix a swap in software. One prop of the wrong hand flips the sign of one column of the allocation matrix: yaw torque and thrust fight each other and the vehicle flips on takeoff. This is the mechanical version of swapping one differential pair.

### Reading the drawing

Open out/frame_drawing.png (generated by hardware/drawing.py). It is a standard two-view drawing. Top view: solid lines are real edges; dashed circles are phantom lines — the swept prop discs, things that exist only in motion; the dotted square is the FC footprint. Thin blue lines with arrowheads are dimensions (46, 65.05, 92, 147 tip-to-tip, 10.05 clearance); a leader with the Ø symbol calls out the 55 mm prop diameter. Spin arrows and circled rotor numbers restate the mixer. Side view: a dimension chain stacks the heights — 7 battery + 3 plate + 16 motor = 26 mm overall. The title block at the bottom is the drawing’s passport: title, date, units, sheet, rev, and — crucially here — the statement matches quadsim/params.py: m = 33 g, L = 46 mm.

## Guided code walkthrough

Open hardware/frame.scad. The header declares its allegiance — “Geometry is BINDING to the simulation model in quadsim/params.py” — and then comes the parameter block:

/* [Core X geometry] */
arm_length = 46;                    // frame center to motor axis (sim L = 0.046 m)
arm_angles = [45, 135, 225, 315];   // deg; index i = sim rotor i+1

/* [Motor press-fit rings — 7x16 mm brushed coreless] */
motor_diam        = 7.0;   // nominal motor can diameter
motor_fit_tol     = 0.1;   // diametral press-fit clearance added to the bore
motor_ring_height = 8;     // grip length on the 16 mm can (must be >= 8)

Every design decision from the theory section is a named number here (the /* [...] */ comments make them sliders in OpenSCAD’s Customizer). Below the parameters come the derived values — never hardcoded, exactly like localparam:

ring_id  = motor_diam + motor_fit_tol;          // press-fit bore
motor_pos = [for (a = arm_angles) arm_length * [cos(a), sin(a)]];
adjacent_spacing = norm(motor_pos[0] - motor_pos[1]);   // = arm_length*sqrt(2)
diagonal_spacing = norm(motor_pos[0] - motor_pos[2]);   // = 2*arm_length

and then the design-rule checks that fail the render if the geometry drifts:

assert(abs(adjacent_spacing - 65.05) <= 0.1, ...);
assert(abs(diagonal_spacing - 92) <= 0.1, ...);
assert(adjacent_spacing - check_prop_diam >= 10, ...);

The geometry itself is three small modules. arm_solid() is one arm laid along local +x — a cube for the arm, a taller thin cube for the stiffening rib, a cylinder for the motor ring — and frame() instantiates it four times with rotate([0, 0, a]), then difference()s away the bores, chamfers, wire slots, FC holes, lightening holes, and the engraved CW/CCW marker dots (1 dot = CCW rotor, 2 dots = CW). Note in arm_cuts() how the lead-in chamfer is cut downward from the ring top — that is the support-free rule showing up in code.

Now docs/DESIGN.md. Its §2 master-dimension table derives everything from the sim (“Arm length … 46.0 mm … params.L = 0.046 m, directly”), and §6 is the weight budget:

| Component | Qty | Unit mass | Subtotal |
| --- | --- | --- | --- |
| 7×16 mm brushed motor | 4 | 3.2 g | 12.8 g |
| 55 mm prop | 4 | 0.4 g | 1.6 g |
| BetaFPV F4 1S brushed AIO FC | 1 | 3.0 g | 3.0 g |
| GNB 300 mAh 1S LiHV | 1 | 7.8 g | 7.8 g |
| Printed frame (PLA) | 1 | 6.0 g | 6.0 g |
| Screws, strap, tape | — | — | 0.9 g |
| All-up total |  |  | 32.1 g |

Its §3 thrust budget puts the spec floor at 16.5 gf/motor → 66 gf total → T/W 2.00 at 33 g, expected ≈ 19 gf fresh (T/W ≈ 2.3) sagging to ≈ 14–15 gf on a tired pack, with hover demanding only 8.25 gf per motor (≈ 43 % throttle).

A cautionary tale — real drift, since reconciled. These documents were written at different times and genuinely drifted before being brought back into agreement; the audit trail is worth studying. Three examples of what the drift looked like: (a) DESIGN.md §4’s parameter table once listed motor_cup_height = 10.0 and arm_width/arm_thickness = 5.5/3.0 while frame.scad actually said motor_ring_height = 8 and 7.5/2.8, under different names; (b) the prop diameter was 55 mm in DESIGN.md, Ø51 in the drawing, and 40 mm in hardware/README.md’s BOM — three documents, three props; (c) the all-up mass totaled 32.1 g (DESIGN.md), 33.0 g (drawing notes), and 34.3 g (hardware/README.md). All four artifacts now agree — 55 mm Hubsan-class prop, 32.1 g budget, frame.scad’s real parameter names — but this is exactly what happens on real teams, and it is why the assert()s live in the geometry file rather than in prose: only the executable checks never drifted.

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026.

Experiment 1 — trace 46 mm end to end. Find the number in all three layers, then verify the derived dimensions:

grep -n "L: float" quadsim/params.py
grep -n "^arm_length" hardware/frame.scad
grep -n "^ARM_L" hardware/drawing.py
.venv/bin/python -c "
from quadsim.params import QuadParams
import math
p = QuadParams()
print('diagonal 2L      =', 2*p.L*1000, 'mm')
print('adjacent 2L/sqrt2=', round(2*p.L*1000/math.sqrt(2), 2), 'mm')
print('d = L/sqrt2      =', round(p.L*1000/math.sqrt(2), 2), 'mm')
print('T/W at 33 g      =', round(p.f_max/p.weight, 3))"

Observe: 46 appears once per layer, each annotated with a comment pointing back at params.py; the derived numbers print 92.0, 65.05, 32.53, 1.977 — matching DESIGN.md §2.

Experiment 2 — regenerate the drawing. .venv/bin/python hardware/drawing.py rewrites out/frame_drawing.png deterministically. Open it and find all five dimensions from Experiment 1 on the sheet. Break it: edit hardware/drawing.py, change PROP_D = 55.0 to PROP_D = 66.0, and rerun. The drawing renders happily — adjacent prop discs now overlap and the CLR callout reads a physically impossible −0.95 — with no error at all. Revert the edit. Lesson: a drawing is a rendering, not a model; it will illustrate a broken design as cheerfully as a good one. Only frame.scad carries assert()s.

Experiment 3 — install OpenSCAD and export an STL. Install (macOS): brew install --cask openscad, or download from openscad.org (need ≥ 2019.05). Then:

/Applications/OpenSCAD-2021.01.app/Contents/MacOS/OpenSCAD -o out/frame.stl hardware/frame.scad

Observe the ECHO: lines: adjacent spacing 65.0538 mm, tip gap 10.05 mm, bore 7.1 mm — the asserts all passed, and out/frame.stl is printable. In the GUI, open hardware/frame.scad, use Window → Customizer to toggle prop_guards = true, press F6, and watch ~5 g of guard rings appear, correctly spaced, without you drawing anything.

Experiment 4 — break the geometry and let the DRC catch you.

/Applications/OpenSCAD-2021.01.app/Contents/MacOS/OpenSCAD -D arm_length=50 -o out/frame_bad.stl hardware/frame.scad

The render fails: adjacent spacing becomes 502=70.71 mm, violating the 65.05 ± 0.1 assert, and OpenSCAD prints the message and refuses to emit an STL. Now try -D motor_fit_tol=0.3 — this one succeeds (bore 7.3 mm) because no assert covers it, and you would only discover the sloppy fit with a motor in hand. Lesson: design rules only protect what someone thought to encode — same as DRC on a board.

## Homework

- Starting from L=46 mm and arms at 45°/135°/225°/315°, derive both the 92 mm diagonal and the 65.05 mm adjacent spacing. Why does the allocation matrix in quadsim/dynamics.py use d=L/2 in its roll/pitch rows instead of L?

Your answer:

- Diff DESIGN.md §4’s parameter table against the actual parameter block in hardware/frame.scad and verify every name and value matches. Before the reconciliation it listed motor_cup_height = 10.0 and arm_width/arm_thickness =    5.5/3.0 — which file should have won each dispute, and what one-sentence policy would prevent the drift from recurring?

Your answer:

- The spec prop is 55 mm; before the reconciliation the drawing said 51 mm and hardware/README.md’s BOM said 40 mm. Check all three against the ≥ 10 mm tip-clearance rule with 65.05 mm spacing. Which choices are legal? What physically happens with a 60 mm prop, and which line of frame.scad would catch it — under what condition?

Your answer:

- DESIGN.md claims T/W = 2.00 at the spec floor, 1.98 at the sim limit, and “≥ 1.83 even in the worst case” at 36 g. Recompute all three. Which thrust number does each claim use, and what do you get at 36 g using f_motor_max instead of the spec floor?

Your answer:

- The frame prints with zero support material. Identify three specific features in frame.scad (quote the module or parameter) that make this true, and explain why the arms-flat orientation is also the strongest orientation for flight loads.

Your answer:

- DESIGN.md §2 claims four 3.2 g motors at d=32.5 mm contribute ≈ 1.35×10−5 kg·m² to Ixx and ≈ 2.7×10−5 kg·m² to Izz (at r=46 mm). Verify both with the point-mass formula and compare against params.J. Why does the motor sit at radius d for roll but L for yaw?

Your answer:

## Hints

- Place the four rotors at (±d,±d) and compute two distances; the torque about the x-axis depends on each rotor’s perpendicular offset from that axis.

- Compare parameter names first — several concepts were renamed — then values; think about which file is executable and which is prose.

- Clearance = spacing − diameter. For the last part, look at check_prop_diam and ask what value it currently holds.

- 16.5 gf and 0.16 N are not the same thrust; 1 gf=9.81 mN.

- Search the SCAD comments for “support-free” and “cut from above”; for strength, recall which direction a laminate is weak.

- A point mass m at perpendicular distance r from an axis contributes mr2; project each rotor position onto the axis in question.

## Further reading

- OpenSCAD User Manual + Cheat Sheet (openscad.org/documentation) — the whole language fits on one page; skim it before Experiment 3 and frame.scad reads like plain code.

- Prusa Knowledge Base, “Design rules for 3D printing” — the practical numbers (overhang angles, hole shrinkage, minimum walls) behind every choice in §Design for 3D printing, from people who print for a living.

- Kushleyev, Mellinger, Powers, Kumar, Towards a swarm of agile micro quadrotors (2013), §II — the vehicle-design section of the reference stack: watch the same mass/inertia/agility budget logic play out on a real 73 g machine.

# Chapter 7 — The real flight stack: where ECE comes home

## After this chapter you can …

- Map every quadsim module to its Crazyflie counterpart, and name the one job reality adds that the sim never had to do.

- Explain IMU attitude estimation: why neither gyro nor accelerometer works alone, how a complementary filter splits the spectrum between them, and what a Kalman filter adds.

- Predict — with numbers from our gains — what estimator bias and control latency do to a hover, and verify both predictions in simulation.

- Describe the Crazyflie architecture (STM32 firmware, radio SoC, cflib, Crazyradio) and locate our controller.py and trajectory.py inside it.

- Make a defensible Level 0→3 purchase decision and recite the safety rules you will actually follow.

## The ECE bridge

This is the chapter where the bridge carries the most traffic, most of it in your direction.

| New concept | What you already know |
| --- | --- |
| Complementary filter | A two-way audio crossover: low-pass one source, high-pass the other, transfer functions summing to 1. The woofer is the accelerometer, the tweeter the gyro |
| Gyro bias drift | DC offset in an amplifier chain: integrate it and it becomes a ramp. Boot-time calibration is auto-zeroing |
| Kalman filter for attitude/position | You know Kalman. Only the application is new: process model = Chapter 2’s dynamics, measurements = IMU/flow/Lighthouse |
| Estimator bias → position offset | Op-amp input offset voltage: a constant input error appears at the output scaled by a gain ratio |
| Control latency instability | Phase margin: a pure delay e−sTd adds no gain but ωTd of phase lag — the loop rings, then oscillates |
| Motor spin-up dynamics | An RC time constant: commanded thrust is a step, delivered thrust a first-order lag (~20 ms) |
| Battery sag | Supply droop under load: available thrust falls as the pack drains |
| PWM brushed-motor drive | H-bridges and duty cycles — home turf |
| 2.4 GHz Crazyradio link | ISM band, GFSK, packet protocols, link budgets — home turf |
| Firmware on an STM32F405 | Embedded C on a Cortex-M4 under an RTOS: a hard-real-time 500 Hz task, not a Python for loop |

## Core theory

### What the simulator gets for free

Open quadsim/sim.py; one line of the loop:

# quadsim/sim.py — simulate()
f_cmd, tau_cmd = controller.compute(state, ref_fn, t)

The controller is handed state — the actual state the integrator produces: exact position, velocity, attitude, instantly, at 500 Hz. No real vehicle has ever known its own state. This line is the biggest lie in the simulator, and the honest name for the missing machinery is state estimation. Everything else reality adds — motor lag, battery sag, wind — is a disturbance, and good feedback loops shrug off disturbances. But a controller can only fight what it can see, and reality makes seeing hard.

### Attitude from an IMU: two bad sensors make one good one

A MEMS IMU gives two measurements. The gyroscope reads angular velocity ω — precisely state.w — and integrating it gives attitude via Chapter 1’s q=12q⊗[0,ω]. But every gyro adds a small bias b, and integrating a constant gives a ramp: attitude error grows as b⋅t, so even a good bias of 0.02 rad/s ruins the estimate within a minute. The accelerometer measures specific force: when the vehicle isn’t accelerating, that is gravity in the body frame — a drift-free reading of down, hence of roll and pitch (never yaw: rotating about gravity doesn’t move gravity). But it is noisy, and it lies whenever the vehicle accelerates.

So one sensor is clean at high frequency and drifts at DC; the other is trustworthy at DC and garbage at high frequency. You solved this with audio drivers: build a crossover with fusion time constant τ,

θ=τsτs+1⏟high-pass⋅ωgyros + 1τs+1⏟low-pass⋅θaccel.

The two filters sum to exactly 1 — no frequency is lost, hence complementary. Discretized at step dt with α=τ/(τ+dt), it collapses to one line:

θk=α (θk−1+ωk dt)+(1−α) θaccel,k.

Choosing τ is a genuine trade: large τ trusts the gyro longer, filtering accel noise hard but letting bias leak through as a steady error of bτ; small τ kills the bias but passes jitter. There is a U-shaped optimum; Experiment 3 finds it.

Now say the magic words: a Kalman filter is a complementary filter that computes τ for you, optimally, every step, from the noise covariances — and, better, it can append b to the state vector and estimate the bias itself, removing the trade instead of balancing it. The Crazyflie firmware runs exactly this — an extended Kalman filter (nonlinear process, hence the E) fusing gyro, accelerometer, and whichever deck is installed into the full [p,v,q] estimate. When docs/HARDWARE.md promises Flow-deck “position hold with no external positioning system,” the enabling technology is that EKF.

### What else reality adds

Motor dynamics. The sim delivers any within-limits thrust instantly. A real coreless brushed motor is an R-L circuit spinning a mass: command a thrust step and it arrives as a first-order lag, time constant 15–25 ms — pure phase lag inside our fastest loop, the attitude loop.

Latency, quantitatively. Sensor read-out, estimator, control, PWM update: each real pipeline stage adds delay Td, contributing phase lag ωTd with no gain change — the classic phase-margin thief. Our attitude loop has kR=1000 per axis on inertia-normalized error, so ωn=1000≈32 rad/s; a 30 ms delay costs 0.03×32≈0.95 rad ≈54° at crossover — most of a healthy margin. Prediction: ringing past 20 ms, departure around 30–40 ms. Experiment 2 checks this.

Battery sag and the missing integrator. A 1S LiPo starts at 4.2 V and sags toward 3.0 V under load; thrust per unit PWM falls with it. Our SE3Controller constructor has kp, kv, kR, kw — no ki anywhere. In sim that is fine: the model is exact, so feedforward plus PD leaves no steady error. In reality, any constant unmodeled force — a draining pack, an off-center battery, a bent prop — leaves a steady offset that only integral action removes. controller_mellinger.c is our controller plus exactly that repair: gains ki_xy, ki_z, and integral attitude terms accumulated in an i_error state — the difference to hunt for in Homework 3.

### The Crazyflie architecture: your sim, compiled

The Crazyflie 2.1+ is two processors and a protocol. An STM32F405 (Cortex-M4, 168 MHz) runs the flight firmware under FreeRTOS; the stabilizer loop ticks at 500 Hz — the same rate as our dt = 0.002 sim loop, because we copied it. An nRF51 handles the 2.4 GHz radio and power management; on your desk a Crazyradio 2.0 dongle speaks the same GFSK physical layer, carrying CRTP packets — a packetized ISM-band link you could characterize in your sleep.

The firmware is our repo in C, module for module — docs/HARDWARE.md tabulates the correspondence:

| Sim module | Real-world counterpart |
| --- | --- |
| quadsim/params.py | Vehicle mass/inertia/thrust limits in the Crazyflie firmware; refine via system ID |
| quadsim/dynamics.py | The physical vehicle (reality is the integrator) |
| quadsim/maths.py | Firmware math libraries (quaternion/rotation utilities) |
| quadsim/trajectory.py | cflib high-level commander; uploaded piecewise-polynomial trajectories (7th-order, same form) |
| quadsim/controller.py | Firmware controller_mellinger.c (select via parameter) |
| quadsim/sim.py | Flight tests; onboard logging via cflib/cfclient log blocks |
| quadsim/swarm.py | Crazyswarm2 (ROS 2): multi-vehicle upload, broadcast start, synchronized execution |
| quadsim/viz.py | cfclient plotter, log-file analysis, rosbag + your own matplotlib scripts |

Read the middle rows. quadsim/dynamics.py maps to the physical vehicle: reality is the integrator, and it has no numerical error. Chapter 3’s controller is selected in firmware via the stabilizer.controller parameter. The trajectory row should make you grin — the firmware’s uploadable format is piecewise 7th-order polynomials in (x,y,z,yaw), and in quadsim/trajectory.py:

# quadsim/trajectory.py
_N_COEFF = 8            # 7th-order polynomial -> 8 coefficients per segment per axis

Same representation, coefficient for coefficient. Flying your Chapter 4 minimum-snap course on hardware is a formatting exercise via cflib’s high-level commander (takeoff, go_to, upload_trajectory, start_trajectory) — the role traj.eval plays for simulate.

### Spending money: the Levels, and when

docs/HARDWARE.md lays out four levels: Level 0, this repo (≈ $0); Level 1, one Crazyflie + Flow deck + Crazyradio (~$360–400), scripted indoor flight with no infrastructure; Level 2, add Lighthouse positioning (~+$450), lab-grade state estimation for aggressive trajectories; Level 3, the Crazyswarm2 multi-vehicle path (~+$350 per vehicle). All prices approximate — check the store first.

The decision framework is a gate, not a shopping list. Move 0→1 only when Level 0 is exhausted: you can read a tracking plot and say which gain to touch, you have broken every demo on purpose, and you can state what the Flow deck cannot give you (drifts over minutes, wants a textured lit floor, gentle speeds, altitude below ~2 m). Move 1→2 only when the flow estimator measurably limits you; 2→3 only with netting up and the formation verified collision-free in sim. At every gate ask: is hardware the bottleneck, or am I buying motivation? The alternatives (~$40–120 tiny whoop for stick skills, an ESP-drone toe-in) exist for honest answers of “motivation.”

### Safety culture

The TED-talk lab flies inside netting, and that lab flies better than you do. From docs/HARDWARE.md: props off for bench work; safety glasses when debugging close-up; replace bent props — vibration wrecks state estimation before it wrecks the frame, and now you know why: vibration is in-band accelerometer noise. LiPos: charge attended on a non-flammable surface, retire puffed or crashed packs, store at ~50%, never fly below ~3.0 V/cell. Net the flight volume before any aggressive or multi-vehicle flight; indoors you set the rules, outdoors aviation law applies. Wire up cflib’s emergency stop before your first scripted flight and test it on the ground. First flights: low, slow, one vehicle, cushion underneath.

## Guided code walkthrough

This walkthrough is short because its point is an absence. In quadsim/controller.py, compute opens with:

# quadsim/controller.py — compute()
R = state.R
w = state.w

ref = ref_fn(t)
e_p = state.p - ref.pos
e_v = state.v - ref.vel

Those reads — p, v, R, w — are the estimator’s job description on hardware. Nothing else changes when you go real: controller_mellinger.c computes the same F_des, eR, and torque; it just receives its state from the EKF instead of the integrator, plus the i_error terms our exact-model sim never needed. And quadsim/sim.py’s History — t, p, v, q, w, f, tau, ref_p, ref_v — is precisely the signal set you will configure as cflib log blocks, so your Chapter 3 plotting habits transfer unchanged. The sim-to-real jump is not a new stack; it is the same stack with one hard new module (estimation) and three soft ones (lag, sag, delay).

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026. Each experiment wraps the real SE3Controller in a fake estimator — degrading what the controller sees while the physics stays perfect: reality’s problems, added one at a time.

Experiment 1 — what kind of error actually hurts? Corrupt the state estimate during an 8 s hover three ways: white noise (a jittery estimator), a constant velocity bias (a miscalibrated flow sensor), and a position random walk (flow drift).

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.sim import simulate
from quadsim.trajectory import hover_ref

p0 = np.array([0.0, 0.0, 1.0])

class Estimator:
    """The controller sees reality through this."""
    def __init__(self, inner, sp=0.0, sv=0.0, vbias=None, walk=0.0, seed=0):
        self.inner, self.sp, self.sv, self.walk = inner, sp, sv, walk
        self.vbias = np.zeros(3) if vbias is None else np.asarray(vbias)
        self.drift = np.zeros(3)
        self.rng = np.random.default_rng(seed)
    def compute(self, state, ref_fn, t):
        self.drift += self.rng.normal(0, self.walk, 3)
        est = QuadState(p=state.p + self.drift + self.rng.normal(0, self.sp, 3),
                        v=state.v + self.vbias + self.rng.normal(0, self.sv, 3),
                        q=state.q, w=state.w)
        return self.inner.compute(est, ref_fn, t)

for name, kw in [("perfect estimate   ", dict()),
                 ("white noise only   ", dict(sp=0.01, sv=0.05)),
                 ("+0.1 m/s vel bias  ", dict(vbias=[0.1, 0, 0])),
                 ("1 mm/step pos drift", dict(walk=0.001))]:
    est = Estimator(SE3Controller(QuadParams()), **kw)
    hist = simulate(QuadParams(), est, hover_ref(p0), QuadState.hover(p0), 8.0)
    final = hist.p[-1] - p0
    print(f"{name} rms={hist.rms_pos_error(2.0)*1000:6.1f} mm   final x offset {final[0]*1000:+7.1f} mm")
EOF

Observe: 1 cm / 5 cm/s white noise costs only ~2 mm RMS — the vehicle’s inertia low-passes zero-mean noise, the same reason a sigma-delta ADC tolerates huge quantization noise. But the 0.1 m/s velocity bias parks the vehicle exactly −50.0 mm off target — predictable: at equilibrium kpep=−kvb, so ep=−(kv/kp) b=−0.5×0.1 m. Input offset voltage, quadrotor edition. The drift case wanders without settling. Moral: estimators are judged by bias and drift, not noise floor — why bias-estimating Kalman filters earn their complexity.

Experiment 2 — latency, and (break it) the phase-margin cliff. Delay the controller’s view of the state by N steps and fly a 30 cm step:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.sim import simulate
from quadsim.trajectory import hover_ref

p0 = np.array([0.0, 0.0, 1.0])
s0 = QuadState.hover(np.array([0.3, 0.0, 1.0]))   # 30 cm step response

class DelayedEstimate:
    def __init__(self, inner, n_steps):
        self.inner, self.buf, self.n = inner, [], n_steps
    def compute(self, state, ref_fn, t):
        self.buf.append(state)
        old = self.buf[0] if len(self.buf) <= self.n else self.buf[-1 - self.n]
        return self.inner.compute(old, ref_fn, t)

for n in [0, 5, 8, 10, 12, 15, 20]:
    ctrl = DelayedEstimate(SE3Controller(QuadParams()), n)
    hist = simulate(QuadParams(), ctrl, hover_ref(p0), s0, 6.0)
    e = hist.pos_error()
    print(f"delay {n*2:3d} ms   peak err {min(np.nanmax(e), 999):7.3f} m   final err {min(e[-1], 999)*1000:9.1f} mm")
EOF

Observe: nothing measurable through 16 ms, a 12 mm residual ring at 20 ms, 71 mm at 24 ms, a 394 mm limit-cycle at 30 ms — and the break-it case, 40 ms, departs to ~100 m: total loss, right where the 54°-of-phase estimate said it would be. The cliff from “fine” to “gone” spans barely 20 ms — which is why flight firmware is hard-real-time C on an RTOS, not Python over USB.

Experiment 3 — build the complementary filter, then (break it) skip gyro calibration. Sixty seconds of synthetic 500 Hz IMU, one axis: true roll is a slow wobble; the gyro has bias 0.02 rad/s plus noise; the accel-derived angle is unbiased but noisy (σ=0.05 rad).

.venv/bin/python - <<'EOF'
import numpy as np
dt, T = 0.002, 60.0
t = np.arange(0.0, T, dt)
true_angle = 0.3 * np.sin(2 * np.pi * 0.5 * t)
true_rate = 0.3 * 2 * np.pi * 0.5 * np.cos(2 * np.pi * 0.5 * t)
rng = np.random.default_rng(1)
bias = 0.02                                   # rad/s — break it: set to 0.2
gyro = true_rate + bias + rng.normal(0, 0.02, t.size)
accel_angle = true_angle + rng.normal(0, 0.05, t.size)

gyro_est = np.cumsum(gyro) * dt
print(f"gyro only   rms {np.sqrt(np.mean((gyro_est - true_angle)**2))*57.3:6.2f} deg (drifts forever)")
print(f"accel only  rms {np.sqrt(np.mean((accel_angle - true_angle)**2))*57.3:6.2f} deg (white jitter)")
for alpha in [0.9, 0.99, 0.999, 0.9999]:
    comp = np.zeros_like(t)
    for k in range(1, t.size):
        comp[k] = alpha * (comp[k-1] + gyro[k] * dt) + (1 - alpha) * accel_angle[k]
    tau = alpha * dt / (1 - alpha)
    err = comp - true_angle
    print(f"alpha={alpha:6.4f} (tau {tau:6.2f} s)  rms {np.sqrt(np.mean(err**2))*57.3:5.2f} deg  "
          f"bias leak ~ {bias*tau*57.3:5.2f} deg")
EOF

Observe: gyro alone drifts to ~39° RMS; accel alone jitters at ~2.9°; the fusion bottoms out at 0.28° at α=0.99 (τ=0.2 s), the U-shape visible on both sides — noise leaks in at small τ, bias (≈bτ, check the printed column) at large τ. Break it: set bias = 0.2, as if the vehicle was moved during boot calibration. The sweet spot collapses toward small τ, the best error more than doubles, and the formerly optimal α=0.99 becomes 8× worse. This is why a Crazyflie must sit still for its first seconds after power-on — and why the EKF, which estimates the bias instead of splitting the difference, earns its arithmetic.

## Homework

- The essay (Module 7’s “check” in docs/LEARNING.md): name the three biggest sim-to-real gaps for flying our minimum-snap course on a Level-1 Crazyflie, and for each, say what symptom you would see in the flight logs — the fingerprint, not just the category.

Your answer:

- Shopping decision: you have $400 and Level 0 is genuinely exhausted. Using docs/HARDWARE.md, choose between (a) the full Level-1 kit, (b) a tiny whoop + radio now, Level 1 later, and (c) an ESP-drone. State your choice, the strongest argument against it, and the observable milestone triggering your next purchase.

Your answer:

- Read one function of real firmware: open controller_mellinger.c in the crazyflie-firmware repository (src/modules/src/controller/), find the main controller function, and identify (i) our F_des line — the position/velocity/feedforward force sum — and (ii) two terms it has that ours lacks, naming which reality each one fights.

Your answer:

- Hands-on, predict then verify: Experiment 1’s 0.1 m/s velocity bias produced a −50 mm offset because ep=−(kv/kp)b. Predict the offset with SE3Controller(QuadParams(), kp=0.033*np.array([32.,32.,32.])) (doubled kp, same kv), then modify the experiment to check. What op-amp fact about loop gain and offsets does this illustrate?

Your answer:

- Show that the complementary filter’s two paths sum to unity — τsτs+1⋅1s⋅s+1τs+1=1 when both sensors read the truth — and say in one sentence why this “all-pass in disguise” property is the whole point.

Your answer:

- Experiment 2 delayed the full state. On real hardware the gyro path (IMU→controller, onboard) has far lower latency than the position path (external system over radio). Which of the four estimated quantities tolerates the least delay, and what does that imply about where the attitude loop must physically run?

Your answer:

## Hints

- One gap is this chapter’s star; the other two are in “What else reality adds.” A fingerprint reads like “altitude sags late in the flight.”

- No letter is wrong, only justifications; answer “what is the current bottleneck?” first.

- Search the file for i_error and gain names starting with ki; our F_des is quadsim/controller.py line 106 — find its C twin.

- The formula answers in one line; the op-amp fact is about what more loop gain does to input-referred offsets.

- The gyro path is (angle × s) → integrator (1/s) → high-pass; multiply it out and add the low-pass.

- Which loop’s crossover (32 rad/s vs 4 rad/s) sets the delay budget, and what does “onboard” mean with two processors?

## Further reading

- Mahony, Hamel & Pflimlin, “Nonlinear complementary filters on the special orthogonal group,” IEEE TAC 2008 — Experiment 3’s filter done properly on SO(3) with Chapter 1’s math; the algorithm inside many hobby flight controllers.

- Mueller, Hamer & D’Andrea, “Fusing ultra-wideband range measurements with accelerometers and rate gyroscopes for quadrocopter state estimation,” ICRA 2015 — the lineage of the EKF the Crazyflie flies; Kalman applied, with Chapter 2’s dynamics as the process model.

- The crazyflie-firmware source (github.com/bitcraze/crazyflie-firmware) — controller_mellinger.c, the EKF in kalman_core, the trajectory player: this repo in C, free to read before you spend a rupee.

- Bitcraze’s “Getting started” and cflib guides (bitcraze.io) — the exact Level-1 bring-up path, including the motor-test and radio-setup rituals the safety section assumes.

Course index · Next: Chapter 8 — Decentralized swarms →

# Chapter 8 — Decentralized swarms: nobody in charge, everybody with the plan

There is a moment in Kumar’s TED talk where the whole lecture pivots. The robots, he says, compute their control a hundred times a second from what they sense from their neighbors — and he puts up a slide with one small equation on it, the pairwise formation error eij=xi−xj−sdes, citing [Turpin, Michael & Kumar 2011]. Two arguments carry the moment. Scale: a central computer coordinating every robot cannot keep up as the swarm grows, so each robot must fly itself from local information. Anonymity: the robots are interchangeable — robot i does not care which robot is next to it, only where it is. Chapter 5 built a nine-robot swarm and then confessed, in its honesty section, how much of it was centralized convenience. This chapter earns part of the claim back: the same nine robots, the same grid → ring → circle-lap show, but the corrective control now uses only relative positions of neighbors within a sensing radius. No robot reads another robot’s absolute state or velocity, ever. The punchline — and it is the Turpin–Michael–Kumar punchline, not a bug — is how little changes.

## After this chapter you can …

- State the information model of decentralized formation flight — what each robot knows, what it senses, and what it never needs — and write Kumar’s slide law eij=xi−xj−sdes in this repo’s notation.

- Recast “mean of pairwise errors” as consensus — Laplacian feedback on slot errors — separate the differential (shape) mode from the common mode, and name which loop owns each.

- Predict from the sensing graph’s connectivity what shrinking Rsense does (a cliff) versus what per-edge dropout does (graceful degradation), and verify both numerically.

- Explain implicit coordination — why nine robots that never exchange a word still agree to sub-millimeter with the centralized swarm, because each carries the same plan.

- Say honestly which parts of this sim remain centralized, which parts of Kumar’s own demos were centralized (Vicon fed every state), and what real relative sensing would take.

## The ECE bridge

| New concept | What you already know |
| --- | --- |
| Consensus (mean-of-neighbors correction) | Distributed clock sync and gossip protocols: every node nudges toward the average of what it hears; there is no master, and the network agrees anyway |
| Pairwise error eij | Differential signaling: the leader’s position cancels out of xi−xj the way common-mode noise cancels off a twisted pair — the correction regulates only the differential mode |
| Sensing-graph connectivity | Network topology: a partitioned network cannot reach consensus, no matter how good each link is; each partition converges to its own value |
| Sensing radius Rsense | Radio link budget: you only hear nodes whose signal clears your receiver sensitivity, and that range — not your intentions — defines the graph |
| Per-edge dropout pdrop | Packet loss: a lossy link lowers throughput but does not move the protocol’s fixed point; retries average it out |
| Implicit coordination via a shared plan | GPS-disciplined oscillators: thousands of them, never exchanging a word, all holding the same 10 MHz — because each independently tracks one broadcast reference |

## Core theory

### The information model

Read the docstring at the top of quadsim/decentralized.py; it is a contract. Robot i knows three things: its own state (position, velocity, attitude — ch07 told you what that costs in reality), the shared leader plan ℓ(t) with the full keyframe schedule, and its own slot offset δi(t) — plus everyone else’s offsets δj(t), since the schedule is common knowledge. It senses one thing: the relative position xi−xj of each neighbor j within Rsense. It never reads a neighbor’s absolute position, velocity, or intent. This is the talk’s information model: the plan is broadcast once (implicit coordination — everyone carries the same score), and everything at runtime is local.

### The pairwise law, and why it is consensus

Kumar’s slide says eij=xi−xj−sdes: the error between where your neighbor actually is, relative to you, and where the formation says they should be. In our notation sdes=δi−δj, so

eij=(xi−xj)−(δi−δj),  ai=−kform 1|Ni|j∈Ni​eij,

with Ni the sensed neighbors and the correction norm-capped at aform,max=3 m/s2, injected into robot i’s reference acceleration exactly the way Chapter 5 injected repulsion.

Now the change of coordinates that makes the structure jump out. Let yi=xi−ℓ(t)−δi — robot i’s error relative to its own slot. Then

eij=yi−yj,

and notice what vanished: the leader position ℓ cancels from every pairwise error. That cancellation is the entire reason relative sensing suffices — the common mode never appears in the measurement, like common-mode noise never appearing across a differential pair. The correction becomes

ai=−kform(yi−1|Ni|j∈Ni​yj),

which is exactly −kform [L y]i with L the row-normalized graph Laplacian of the sensing graph. This is the consensus protocol of Olfati-Saber, Fax & Murray: each node relaxes toward the average of its neighbors, like a grid of resistors relaxing toward one voltage. On a connected graph, L’s null space is spanned by the all-ones vector: the only error the pairwise terms cannot see is everyone off by the same amount — a rigid translation of the whole formation. And that one mode is precisely what the shared plan’s absolute position loop (Chapter 3’s kpep, running on each robot’s own state) regulates. Clean division of labor: consensus kills the shape error, the plan kills the common mode. Differential and common-mode rejection, flying in formation.

One gain choice matters. The docstring sets kform=4 s−2, a quarter of the position-loop stiffness kp/m=16 s−2, and gives the reason: the pairwise term is a spring on relative position with no relative damping — there is no relative-velocity measurement to damp it. The only damping the relative modes get is second-hand, through each robot’s own kv loop, so the added spring must stay soft. Homework 5 has you harden it and watch.

### Connectivity is the convergence condition

Everything above assumed the sensing graph is connected. Suppose it is not — Rsense too small, or the formation too spread out. The Laplacian’s null space grows: one all-ones vector per connected component. Within each component, robots still agree on shape; between components, there is no measurement at all. Each component then flies the shared plan dead-reckoned — open loop with respect to the other components, closed only on its own state estimate. In this simulator that failure is disguised, because every robot knows its own state perfectly, so each component independently parks on its absolute slots anyway (Experiment 3 shows the residual coupling collapse). On hardware, a disconnected component holds formation only as well as its estimator holds position — and Chapter 7 told you what flow-deck drift does over minutes. The graph is also state-dependent: it is actual distances, not slot distances, that decide who senses whom, so a formation can disconnect mid-transient even when its slots are fine. Connectivity is a design deliverable, exactly like network topology: the demo chose Rsense=1.2 m as roughly twice the nearest-neighbor slot spacing (0.55 m grid, 0.62 m ring chord) so the graph stays connected through the blend — yet local, since on the ring only the 0.62 m and 1.16 m chords are in range and the 1.56 m chord is not.

### Honesty: what is still centralized — here, and in the videos

The control is now decentralized: robot i’s corrective term uses its own state plus sensed relative positions, nothing else. Still centralized in our sim: the slot assignment is solved once, offline, by Chapter 5’s Hungarian machinery — anonymity, the second half of Kumar’s argument, is exactly the assignment problem, and pre-assigning δi quietly de-anonymizes the robots (Homework 2). The plan distribution is perfect: every robot has bit-identical keyframes and a flawless clock (implicit coordination assumes the broadcast worked). And the relative positions are ground truth from the simulator, delivered as full 3-D vectors in a common frame — real relative sensing gives you a scalar range (UWB) or a bearing (camera), in your own body frame, with noise (Homework 3). Before you feel bad about any of this: Kumar’s famous lab demos ran decentralized control laws while a Vicon motion-capture system measured every vehicle’s state centrally and streamed it down — the same honesty, at higher production values. In Kushleyev et al. (2013), only the kilohertz attitude loops ran onboard. The demos that finally closed the loop — onboard sensing, onboard planning, no mocap, outdoors — took another decade: the 2022 Zhejiang “wild swarm” (Further reading 4).

## Guided code walkthrough

Open quadsim/decentralized.py. The whole chapter is one subclass:

class DecentralizedSwarmSim(SwarmSim):

Keyframe validation, Hungarian slot assignment, smoothstep blending, and pairwise repulsion are all inherited from Chapter 5 unchanged. The constructor adds R_sense, k_form, a_form_max, and the dropout pair p_drop/seed, and enforces one new invariant — d_safe <= R_sense — because the inherited repulsion also runs on relative positions of nearby pairs, and a safety reflex against threats you cannot sense is not a safety reflex (Experiment 5 trips this guard).

The new control law is formation_accels. Its core is four lines:

diff = positions[:, None, :] - positions[None, :, :]
dist = np.linalg.norm(diff, axis=2)
sensed = dist <= self.R_sense
np.fill_diagonal(sensed, False)

— the sensing graph, rebuilt from actual distances every step (state-dependent, as promised), with each directed edge then independently surviving a rng.random(...) >= self.p_drop draw when dropout is on. Then, per robot:

err = diff - (off[:, None, :] - off[None, :, :])
...
a_i = -self.k_form * err[i, idx].mean(axis=0)

That is eij=(xi−xj)−(δi−δj) and the mean-of-neighbors correction, norm-capped at a_form_max. A robot that senses nobody gets zero correction — it flies the plan and nothing else, which is the disconnection story of the theory section made executable. The offsets come from blended_offsets(t), so the law tracks formation transitions mid-blend.

run mirrors SwarmSim.run with one changed line:

a_corr = (self._avoidance_accels(positions)
          + self.formation_accels(positions, t, rng))

Both injected terms ride into _formation_ref held constant in t — the Chapter 5 lesson about not detonating the finite-difference attitude feedforward applies verbatim. Two additions earn their keep in the experiments: start_offsets lets you launch the swarm off its slots (without it, eij≈0 from the first step and there is nothing to watch), and the dropout generator is rebuilt from seed on every run call, so lossy runs are bit-identical and diffable — tests/test_decentralized.py::test_dropout_determinism pins that down. Finally, relative_formation_errors is the chapter’s own metric: the per-sample RMS of ∥eij∥ over all pairs — pure formation shape error, blind to absolute leader tracking, which is exactly the quantity the decentralized term regulates.

demos/demo_decentralized.py flies the same ~11 s show three ways — SwarmSim (centralized baseline), DecentralizedSwarmSim, and DecentralizedSwarmSim with p_drop = 0.3 — and prints the same four metrics for each — the three error metrics at six decimals, because the centralized-vs-decentralized gap is sub-millimeter by design. The comment block above R_SENSE = 1.2 does the connectivity arithmetic from the theory section with the demo’s actual geometry; read it.

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026.

Experiment 1 — baseline: three swarms, one dance.

.venv/bin/python demos/demo_decentralized.py --fast

(~29 s.) Observe the three variants side by side. Centralized: steady_formation_rms_m 0.000011, max_formation_error_m 0.000726, min_pairwise_distance_m 0.4102. Decentralized: 0.000004, 0.000710, 0.4102. Decentralized with 30 % dropout: 0.000004, 0.000710, 0.4102 — indistinguishable from lossless at this precision. The swarm that senses only neighbors within 1.2 m matches — fractionally beats — the swarm with perfect central knowledge, and the relative-shape RMS (relative_rms_m 0.000006 vs the centralized 0.000016) shows why: the consensus spring actively regulates shape, which the centralized baseline never measures. This agreement is the Turpin–Michael–Kumar point: with implicit coordination, every robot executing the same plan is already almost exactly where its neighbors expect, so the sensed correction idles near zero. Relative sensing is a trim tab here, not the pilot — until something goes wrong, which is what the rest of the experiments are for.

Experiment 2 — the full show.

.venv/bin/python demos/demo_decentralized.py --no-gif

(~55 s; drop --no-gif if you want decentralized.gif.) The full 11 s timeline adds the leader’s circle lap. All three variants land on steady_formation_rms_m 0.000197 — identical to six decimals — with max errors 0.000836 / 0.000815 / 0.000816 m and min pairwise distance 0.4102 m across the board. Steady relative RMS: 0.000003 m centralized, 0.000001 m for both decentralized runs. Yes, those are microns; a perfect model, perfect state, and no disturbances will do that (Chapter 7 priced what reality adds). The number that matters is not any single value but the spread between variants: tens of micrometers. Look at out/decentralized_3d.png — you cannot tell which architecture flew it. That is the point.

Experiment 3 — break it: shrink Rsense to the disconnection cliff. Four robots on a diamond (0.52=0.7071 m between adjacent slots, 1.0 m across), launched deliberately off their slots, with a hovering leader. Sweep Rsense down, and include a k_form = 0 run — the correction surgically removed — as the plan-only reference:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.decentralized import DecentralizedSwarmSim
from quadsim.swarm import FormationKeyframe
from quadsim.trajectory import hover_ref

OFFSETS = 0.5 * np.array([[1.0,0,0],[0,1.0,0],[-1.0,0,0],[0,-1.0,0]])
START = np.array([[0.3,0.1,0.0],[-0.1,-0.2,0.1],[0.0,0.15,-0.1],[0.1,0.0,0.05]])
LEADER = np.array([0.0,0.0,1.0])
keyframes = [FormationKeyframe(t_start=0.0, t_blend=0.1, offsets=OFFSETS.copy())]

slots = LEADER + OFFSETS
dist = np.linalg.norm(slots[:,None,:]-slots[None,:,:], axis=2)
print("slot spacings: adjacent %.4f m, opposite %.4f m" % (dist[0,1], dist[0,2]))

for R, kf in [(5.0,4.0),(5.0,0.0),(1.1,4.0),(0.9,4.0),(0.71,4.0),(0.70,4.0),(0.5,4.0),(0.35,4.0)]:
    sim = DecentralizedSwarmSim(QuadParams(), hover_ref(LEADER), keyframes, R_sense=R, k_form=kf)
    hist = sim.run(T=4.0, dt=0.004, start_offsets=START.copy())
    rel = sim.relative_formation_errors(hist)
    tr = hist[0].t <= 1.5
    n_edges = int(((dist <= R) & (dist > 0)).sum()) // 2
    print(f"R_sense={R:5.2f} k_form={kf:.1f} edges_at_slots={n_edges} "
          f"transient_rel_mean_m={float(rel[tr].mean()):.4f} final_rel_m={float(rel[-1]):.2e}")
EOF

(~23 s.) Observe three things. First, the graph: 6 edges at Rsense=5.0 and 1.1 (complete), 4 at 0.9 (adjacent pairs only), and a hard drop to 0 between 0.71 and 0.70 — the adjacent spacing is 0.7071 m, and connectivity is a threshold, not a dial. Second, the transient shape error degrades from 0.0875 m (complete graph) through 0.0903 and 0.1015 down to 0.1143 m with no sensing — a 1.31× penalty, the same ratio the test suite pins — and the no-sensing run reproduces the k_form = 0 plan-only run exactly (0.1143 m transient, 5.91e-06 m final, digit for digit): a disconnected robot and a robot with no correction term are the same robot. Third, the subtlety: Rsense=0.71 and 0.70 print the same transient error (0.1015 m) despite sitting on opposite sides of the slot-graph cliff, because during the perturbed transient it is actual distances that decide the graph — but their final residuals differ by ≈21× (2.12e-07 vs 4.53e-06 m): once settled near the slots, one swarm is still consensus-coupled and the other is four strangers independently dead-reckoning the same plan. In sim, with perfect self-knowledge, dead reckoning still parks on the slots. On hardware, that final column is your estimator drift.

Experiment 4 — dropout is graceful, disconnection is a cliff. Same diamond, full connectivity, now with lossy sensing:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.decentralized import DecentralizedSwarmSim
from quadsim.swarm import FormationKeyframe
from quadsim.trajectory import hover_ref

OFFSETS = 0.5 * np.array([[1.0,0,0],[0,1.0,0],[-1.0,0,0],[0,-1.0,0]])
START = np.array([[0.3,0.1,0.0],[-0.1,-0.2,0.1],[0.0,0.15,-0.1],[0.1,0.0,0.05]])
LEADER = np.array([0.0,0.0,1.0])
keyframes = [FormationKeyframe(t_start=0.0, t_blend=0.1, offsets=OFFSETS.copy())]

for p in [0.0, 0.3, 0.6, 0.9, 0.99]:
    sim = DecentralizedSwarmSim(QuadParams(), hover_ref(LEADER), keyframes,
                                R_sense=5.0, p_drop=p, seed=0)
    hist = sim.run(T=4.0, dt=0.004, start_offsets=START.copy())
    rel = sim.relative_formation_errors(hist)
    tr = hist[0].t <= 1.5
    print(f"p_drop={p:4.2f} transient_rel_mean_m={float(rel[tr].mean()):.5f} "
          f"final_rel_m={float(rel[-1]):.2e}")
EOF

(~15 s.) Observe: 0.08750 m lossless; 0.08783 at 30 % loss — a 0.4 % penalty, essentially free, which is why the demo’s dropout variant matched the lossless one to six decimals; 0.09174 at 60 %; 0.10573 at 90 %; and 0.11336 at 99 %, converging on the no-sensing value (0.1143 m) from Experiment 3. No cliff anywhere — packet loss, not partition. The reason is structural: the correction is a mean over surviving edges, and every surviving eij still points toward the correct shape, so dropout thins the estimate without biasing it — the fixed point never moves, only the effective gain sags. Losing packets slows a gossip protocol; cutting the cable partitions it. Homework 4 puts probability on exactly where this curve bends.

Experiment 5 — break it: promise a reflex you cannot see. Ask for Rsense=0.2 m with the default dsafe=0.30 m bubble:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.decentralized import DecentralizedSwarmSim
from quadsim.swarm import FormationKeyframe
from quadsim.trajectory import hover_ref
OFFSETS = 0.5 * np.array([[1.0,0,0],[0,1.0,0],[-1.0,0,0],[0,-1.0,0]])
keyframes = [FormationKeyframe(t_start=0.0, t_blend=0.1, offsets=OFFSETS)]
DecentralizedSwarmSim(QuadParams(), hover_ref(np.array([0.,0.,1.])), keyframes, R_sense=0.2)
EOF

Observe: ValueError: d_safe must not exceed R_sense: repulsion is computed from sensed relative positions — at construction, before anything flies. Chapter 5’s repulsion needs a relative position to push along; in the decentralized information model that position must be sensed, so a safety bubble wider than the sensing radius would be a promise the sensors cannot keep. Same habit as Chapter 5’s overlapping-blend guard: encode the physical assumption as a boundary check and fail loudly. The safety implication is worth saying out loud — in this architecture, collision avoidance degrades with the same sensor that formation keeping does; a blind robot is not just sloppy, it is unsafe.

## Homework

- Derive the consensus form: starting from ai=−kform meanj∈Ni eij, substitute yi=xi−ℓ−δi, show the leader cancels, and write the stacked dynamics as a=−kform(I−D−1A) y. What is the null space of that matrix on a connected graph, which physical motion does it correspond to, and which term of Chapter 3’s controller pins it?

Your answer:

- Kumar’s second argument was anonymity — robots should be interchangeable. Our robots are not: each is constructed with a pre-assigned δi. Explain why genuine anonymity turns the offsets into a runtime decision, why that decision is exactly Chapter 5’s assignment problem, and which property of the squared-distance CAPT assignment you would refuse to give up when solving it online.

Your answer:

- Forward-looking: the sim hands each robot xi−xj as an exact 3-D vector in a common frame. What would a real Crazyflie need to measure that? Consider UWB two-way ranging (a scalar distance) and an onboard camera (a bearing in the body frame): what does each alone fail to give you, what does combining them require, and what supplies the shared yaw reference without which “the neighbor is 0.6 m north of me” is not even a well-formed sentence?

Your answer:

- In the Experiment 4 diamond every robot has three potential neighbors and each directed edge drops independently with probability p. Derive the probability that a robot is completely blind in a given step, evaluate it at p=0.3, 0.6, and 0.9, and use those three numbers to explain the shape of the measured sweep — in particular why 30 % loss cost only 0.4 %.

Your answer:

- Hands-on: the docstring keeps kform=4 s−2, a quarter of kp/m=16 s−2, because the pairwise spring adds no relative damping. Predict what kform=64 does to the transient, then rerun Experiment 3’s Rsense=5.0 case with it and report. Where would relative damping have to come from, and what would the robot need to sense to provide it?

Your answer:

- The demo claims Rsense=1.2 m keeps ring sensing “genuinely local.” Compute all four distinct chord lengths of the 9-slot, r=0.9 m ring (2rsin(kπ/9) for k=1..4), count each robot’s neighbors at Rsense=1.2 and at 2.0, and argue both sides: what improves as the graph densifies toward complete, and what — in radio-link-budget terms and in Kumar’s scaling argument — you pay for it.

Your answer:

## Hints

- meanj(yi−yj)=yi−1|Ni|j​yj; stack rows. The null vector has every component equal. The pinning term is the one that reads absolute position error ep.

- If nobody owns a slot in advance, someone must choose at runtime who takes which — from relative information. The property to keep is the one Chapter 5 proved with the parallelogram law: non-crossing straight-line transitions.

- A range alone puts the neighbor on a sphere; a bearing alone on a ray. Think about what rotates a body-frame bearing into a shared frame — and which sensors from Chapter 7 (magnetometer? UWB anchors? Lighthouse?) can anchor yaw.

- Blind means all three incoming edges drop at once: p3. Compare 0.33 with 0.93 and look at which sweep rows bend.

- A spring with stiffness 4× the position loop’s, damped only through each robot’s own kv acting on absolute velocity — expect relative-mode ringing. Watch relative_formation_errors oscillate rather than decay cleanly. Damping it directly would need relative velocity, which the information model deliberately excludes.

- 2×0.9×sin(π/9)≈0.62 m is the first chord; three more to compute. At Rsense→ complete graph, the mean over neighbors approaches the centralized average — good for stiffness, but every added edge is a link your radio/vision must actually sustain, on every robot, at 100 Hz.

## Further reading

- Turpin, Michael & Kumar — “Trajectory design and control for aggressive formation flight with quadrotors” (2011). The citation on the TED slide: the pairwise-error control law this module implements, with the stability analysis and the aggressive-flight results the talk shows off.

- Olfati-Saber, Fax & Murray — “Consensus and cooperation in networked multi-agent systems” (Proceedings of the IEEE, 2007). The consensus survey: graph Laplacians, connectivity as the convergence condition, convergence rate as algebraic connectivity — the theory this chapter used with the serial numbers filed off.

- Kushleyev, Mellinger, Powers, Kumar — “Towards a swarm of agile micro quadrotors” (2013). Reread it after this chapter with one question: for each subsystem, did this run onboard or on the base station? The answers are the honest version of the highlight reel.

- Zhou et al. — “Swarm of micro flying robots in the wild” (Science Robotics, 2022). The Zhejiang University swarm: palm-sized quadrotors flying through a bamboo forest with onboard vision, onboard planning, and onboard relative localization — no mocap, no GPS, no shared plan even. The distance between Chapter 8 and that paper is a fair map of the decade of research between Kumar’s talk and now, and the natural “what’s next” if this chapter hooked you.

← Previous: Chapter 7 — The real flight stack · Course index · Next: Chapter 9 — Your own firmware →

# Chapter 9 — Your own firmware: our controller on the real robot

## After this chapter you can …

- Explain what buying a Crazyflie actually buys — motors, IMU, radio, and a solved hard-real-time substrate — and what this chapter replaces: the controller in the 500 Hz loop.

- Name the crazyflie-firmware extension points (pluggable controllers, the app layer, out-of-tree builds) and say why you write inside the stabilizer loop rather than around it.

- Map every function of quadsim/controller.py onto its counterpart concept in controller_mellinger.c, and state the two structural differences you must verify before trusting the map.

- Write the core SE(3) math in firmware-discipline C — float-only, no malloc — and prove it against golden vectors exported from our Python controller, on your laptop, today.

- Recite the four validation gates between “it compiles” and “it flies”, and state exactly where the flight-test campaign re-enters: FT-1.1, for any custom firmware.

A word on expectations before we start. Everything in this chapter up to and including compiling the C port and passing its unit gate runs today, with no hardware — that is Experiments 1–3. Flashing and flying wait for the vehicle. And flight/README.md Part A already told you when this project becomes a good idea: “Writing firmware becomes a great project the moment you can already fly and read a tracking log, because then you can tell your bug from physics.” This chapter is the roadmap for that moment — walked as far as a laptop allows, so the day the parcel arrives you flash code that has already survived three gates.

## The ECE bridge

| New concept | What you already know |
| --- | --- |
| Pluggable firmware controller | Implementing a fixed callback against a vendor HAL: the RTOS owns the schedule and the drivers, you own the math inside one function |
| 500 Hz stabilizer tick | Worst-case-execution-time budgeting for an ISR: 2 ms per tick, shared with the estimator — blow the budget and the loop slips, and ch07 Experiment 2 priced what slip costs |
| Float-only math | Single-precision DSP discipline: the STM32F405’s FPU (Cortex-M4F) does float in hardware; a stray double literal drops you into software emulation |
| No malloc, static allocation | Embedded C you already practice: all state in static structs, stack bounded, nothing allocated after boot |
| Golden test vectors | HDL testbench vectors: same stimulus into two implementations, diff the outputs bit-for-bit-ish |
| Log replay validation | Hardware-in-the-loop with recorded stimulus: play a captured trace through the new block before it touches the plant |
| Props-off bench test | Bringing up a new board behind a current-limited supply: full power path exercised, no energy to do damage |
| Re-entry at FT-1.1 | Re-qualification after an engineering change: new implementation ⇒ the acceptance tests run again, from the bottom |
| Flashing over the radio | A bootloader/DFU cycle, with the stock image as your known-good ROM: one reflash from safety |

## Core theory

### Why buy a robot and then replace its brain

Chapter 7 made the case for not writing firmware on day one, and nothing here retracts it: the Bitcraze stack’s drivers, EKF, radio protocol, and failsafe stay exactly as they are. What changes is one plug-in. Buying the Crazyflie was buying motors, an IMU, and a radio in a convenient, crash-tolerant arrangement — plus years of solved hard-real-time problems. The stock controller_mellinger.c is a fine brain, and per docs/ARCHITECTURE.md §2 it is what Level-1 flights use. But the whole point of this project was to own the algorithms, and ownership has a final exam: the day quadsim/controller.py — our controller, our gains, our attitude-error convention — commands the real motors.

### The firmware landscape

The flight firmware is open source: github.com/bitcraze/crazyflie-firmware, C on FreeRTOS, with the stabilizer loop ticking at 500 Hz on the STM32F405 — the same rate as our dt = 0.002 sim loop (flight/README.md Part A). Controllers are pluggable: the stock tree ships several under src/modules/src/controller/ — controller_pid.c (the default), controller_mellinger.c, controller_indi.c, and in recent trees more — selected at runtime through the stabilizer.controller parameter (Mellinger is value 2 in current firmware, flight/README.md §C.5). Each one implements the same small interface: an init, a self-test, and a per-tick function that receives the setpoint, the sensor data, and the estimated state, and fills in the motor-facing control output. The exact signatures live in the firmware’s controller interface header (src/modules/interface/controller.h) — read that header rather than trusting any book, this one included: the API has evolved and will again.

Two supported extension points matter to us, both documented on the Bitcraze firmware pages (see the docs’ “Out-of-tree build” and “App layer” chapters in the crazyflie-firmware documentation):

- Out-of-tree controllers: your controller compiles in its own directory against the firmware tree and registers as the selectable controller, without patching Bitcraze’s files. This is the intended home for our port — upstream merges stay trivial.

- The app layer: a sandboxed FreeRTOS task for higher-level logic (sequencing, logging, supervisors). Useful later; it is not where a 500 Hz controller belongs.

The discipline the substrate imposes is the ECE bridge’s right-hand column: your per-tick function must finish comfortably inside 2 ms alongside the EKF, use single-precision floats (the M4F FPU does not accelerate double — write 1.0f, use sinf/sqrtf), and allocate nothing: every accumulator and scratch matrix is a static or a stack array, sized at compile time. Our controller is a few hundred floating-point operations per tick (Homework 4 makes you count them), so the budget is generous — the discipline is about guarantees, not speed.

### The mapping table: controller.py ↔ controller_mellinger.c

Chapter 7’s Homework 3 sent you into controller_mellinger.c hunting for i_error and ki_*; this table is that homework grown up. Left column: our code, with line numbers you can check because the file is in this repo. Right columns: the concept to find in the firmware — deliberately not line numbers, because that file changes; Experiment 4 has you fill the Verify column against today’s source.

| Ours (quadsim/controller.py) | Concept | Find in controller_mellinger.c | Verify |
| --- | --- | --- | --- |
| F_des = -kp*e_p - kv*e_v + m*g*e3 + m*ref.acc (line 106) | Outer loop: desired world force | Position + velocity error terms, gravity, acceleration feedforward — plus integral terms ours lacks (ki_* gains, i_error accumulators; ch07 §“Battery sag”) | What does each i_error integrate, and where is it clamped (anti-windup)? |
| f = max(0, F_des @ (R @ e3)) (line 107) | Thrust = projection of desired force on the actual body z | The scalar thrust output. Units differ: ours is newtons; the firmware’s output is scaled for the power-distribution/PWM stage | Find the scale factor and where saturation happens |
| flat_to_rotation(F_des, ref.yaw) (lines 21–47) | Desired attitude from a force vector + yaw | Construction of desired body axes from desired acceleration and yaw | How does it handle the degenerate heading case our lines 42–44 guard? |
| e_R = 0.5*vee(R_cmd.T @ R - R.T @ R_cmd) (line 120) | SO(3) attitude error | The attitude-error computation. The convention may differ — the Mellinger paper uses this skew-symmetric form, but the firmware implementation may build roll/pitch/yaw errors from body-axis cross products with its own sign conventions | Hand-check one tilt case (Experiment 3) against the firmware’s formula before trusting signs |
| e_w, w_ff, a_ff by finite-differencing ref_fn (lines 110–121) | Rate error + attitude feedforward | Rate error against setpoint-supplied rates. No finite differencing: the firmware receives one setpoint sample per tick, not a callable ref_fn(t) | Which setpoint fields (rates, acceleration) does it actually consume? |
| tau = J @ (-kR*e_R - kw*e_w) + w×Jw - J@(…) (lines 122–126) | Torque, gains premultiplied by J so kR, kw read as ωn², 2ζωn (docs/ARCHITECTURE.md §2) | Attitude gains applied without our J premultiplication, tuned as raw numbers; gyroscopic/feedforward terms may be absent or folded elsewhere | Compare term by term; note which of our terms have no twin |
| kp, kv, kR, kw defaults (lines 72–75) | Pole placement, ζ = 1 everywhere | A different gain set with different names and units | Do not copy numbers. docs/ARCHITECTURE.md §2: the twin is structural; gains never transfer numerically |

The two differences worth reading twice: the firmware has integral action (real batteries sag, real frames trim off-center — ch07 explained why the exact-model sim never needed it), and its attitude-error convention is not guaranteed to be ours — a sign or transpose difference there is not a small bug, it is positive feedback on attitude. Experiment 3 shows what that looks like in numbers.

### The C port, sketched

Here is the heart of the port in firmware-discipline C — the same math as controller.py lines 104–127, float-only, no allocation. (R is row-major float[9]; the full compilable file, including the vector helpers and a test harness, is written by Experiment 2.)

/* e_R = 0.5 * vee(R_cmd^T R - R^T R_cmd).  The argument is exactly
 * skew-symmetric, so vee reads the off-diagonal entries directly. */
static void attitude_error(const float r_cmd[9], const float r[9], float e_r[3]) {
    float m[9];
    mat3_at_b(r_cmd, r, m);              /* m = R_cmd^T R; R^T R_cmd = m^T */
    e_r[0] = 0.5f * (m[3 * 2 + 1] - m[3 * 1 + 2]);
    e_r[1] = 0.5f * (m[3 * 0 + 2] - m[3 * 2 + 0]);
    e_r[2] = 0.5f * (m[3 * 1 + 0] - m[3 * 0 + 1]);
}

/* Outer loop force, thrust projection, torque — controller.py's compute(),
 * hover/setpoint case (w_ff = a_ff = 0; see the walkthrough). */
for (int i = 0; i < 3; ++i)
    f_des[i] = -KP[i] * (p[i] - ref_pos[i]) - KV[i] * (v[i] - ref_vel[i])
             + MASS_KG * ref_acc[i] + (i == 2 ? MASS_KG * GRAV : 0.0f);
re3[0] = r[2]; re3[1] = r[5]; re3[2] = r[8];       /* R e3 = third column  */
f = dot3(f_des, re3);
if (f < 0.0f) f = 0.0f;                            /* thrust cannot pull   */
flat_to_rotation(f_des, ref_yaw, r_cmd);
attitude_error(r_cmd, r, e_r);
cross3(w, jw, wxjw);                               /* gyroscopic term w×Jw */
for (int i = 0; i < 3; ++i)                        /* e_w = w when w_ff = 0 */
    tau[i] = J_DIAG[i] * (-KR[i] * e_r[i] - KW[i] * w[i]) + wxjw[i];

Three notes, one per discipline. Precision: every constant carries an f suffix and every libm call is the f variant (sinf, sqrtf, cosf) — on the M4F, sqrtf is one hardware instruction while sqrt is a software subroutine. Time: this is on the order of a hundred floating-point operations plus a handful of square roots and two trig calls — microseconds against a 2 ms tick (Homework 4 puts numbers on it); the budget threat is never this math, it is accidentally blocking (a print, a log, a semaphore) inside the tick. Memory: gains and inertia are static const tables, scratch lives on the stack with compile-time sizes; the only mutable state a full port adds is integral accumulators — static float, clamped.

### The validation ladder — four gates between “compiles” and “flies”

This is the chapter’s crown, and it is a ladder you already believe in from the flight-test plan: a phase may not begin until every test of the previous phase has passed (docs/FLIGHT_TEST_PLAN.md §1.1). We simply extend the ladder downward, below the bench, all the way to your laptop. Each rung is a gate: do not climb past a rung you have not passed.

Gate 1 — unit level, on the laptop (today). Compile the C math with your desktop compiler and run it against golden vectors exported from quadsim/controller.py: a file of (state, reference) → (f, τ) cases the Python controller has already answered. Pass bar: worst |C − Python| disagreement at float precision (~1e-7 on our magnitudes), across cases that include asymmetric attitudes — Experiment 3 shows why symmetric cases can hide a fatal sign bug. Experiments 1–2 are this gate, executed.

Gate 2 — closed-loop replay, on the laptop (after first stock-firmware flights). Feed logged real-flight states through both implementations and diff the commands. The --log CSVs from flight/fly_hover.py carry t, x, y, z, vbat (flight/README.md §C.4) — enough to start; a fuller replay wants velocity, attitude, and rates from cfclient log blocks (Homework 5 designs the block). This gate catches what unit vectors cannot: real signal magnitudes, real noise, real trajectories — the C port must produce the same commands as controller.py on data no one hand-picked.

Gate 3 — props-off bench, on the vehicle (hardware day). Flash the port; props stay off. Re-run the bench ladder under the new brain: flight/preflight.py (FT-0.1–FT-0.5) and the FT-0.6 emergency-stop drill — the firmware you just flashed is precisely the component the stop path runs through, so the drill is not a formality. Then hold the vehicle level in your hand (props off), select your controller via stabilizer.controller, and watch motor commands respond sanely to tilts: tilt left ⇒ left-side motors speed up. Keep the stock firmware image on disk — one radio-bootloader reflash from safety at all times.

Gate 4 — re-enter the flight-test campaign at FT-1.1. Flying any custom firmware — a full controller port or a one-line patch — re-enters the campaign at FT-1.1 (first hover, 0.5 m / 5 s) and climbs the phases in order, per docs/FLIGHT_TEST_PLAN.md. A new brain is a new configuration, and the hard-gate rule exists exactly for configuration changes; your FT-2 and FT-3 passes under stock firmware say nothing about the code you flashed yesterday. G-DRY, G-PREFLIGHT, and G-BATTERY apply unchanged. No new FT IDs exist for custom firmware — it is the same campaign, re-flown, and the comparison of your FT-2.2 tracking numbers against the stock Mellinger’s is the project’s graduation plot.

## Guided code walkthrough

Read compute() in quadsim/controller.py one more time — this time as a port checklist, sorting each line into “ports directly” versus “must be restructured”.

Ports directly. The state reads (p, v, R, w — in firmware these arrive from the EKF via the state argument, ch07’s whole story), the F_des sum (line 106), the thrust projection and its max(0, ·) clamp (line 107), flat_to_rotation including its degenerate-heading guard (lines 36–44), e_R (line 120), and the torque assembly (lines 122–126). This is the code Experiment 2 compiles, and it is most of the file.

Must be restructured. Everything touching ref_fn. Our Python controller receives the reference as a callable and evaluates it at three times — R_ff(t), R_ff(t ± fd_dt) (lines 110–118) — to finite-difference the attitude feedforward w_ff, a_ff. Firmware receives one setpoint sample per tick; there is no function to probe at t ± 1e-3. The honest options: (a) set w_ff = a_ff = 0 — exactly right for hover and gentle setpoints, since for hover_ref the commanded rotation is constant and the feedforward is identically zero (that is why Gate 1’s golden vectors are hover-referenced, and why our C se3_feedback is complete for that case); (b) for trajectories, consume the rates and accelerations the onboard trajectory player provides in the setpoint — check which fields the setpoint struct actually carries in the firmware headers before designing this; (c) keep a one-tick history onboard and finite-difference across ticks — workable, but you are differentiating a 500 Hz sampled signal and ch07 taught you what that does to noise. Start with (a); flying our full min-snap feedforward is a Gate-2-then-Gate-4 project of its own.

One absence to notice, again: no integral terms — line 106 has four terms and none is an accumulator. The sim’s exact model never needed one; the real vehicle does (ch07 Experiment 1’s bias offset, battery sag). A flyable port grows ki terms exactly where controller_mellinger.c has them, with clamps. Homework 2 makes you write those three lines.

## Experiments

Run everything from /Users/vishalbharti/Downloads/micro-agile-robot-2026. No hardware, no radio, no cflib — a Python interpreter and any C compiler (cc on macOS).

Experiment 1 — export golden vectors from the Python controller. Six hand-chosen cases: perfect hover, a position offset, a descent, a pure tilt, tilt plus body rates, and everything at once with a yaw setpoint. Each row of the CSV is the full controller input (state, reference) and the Python controller’s answer (f, τ) at full precision:

.venv/bin/python - <<'EOF'
import numpy as np
from quadsim.params import QuadParams
from quadsim.controller import SE3Controller
from quadsim.dynamics import QuadState
from quadsim.maths import quat_normalize
from quadsim.trajectory import hover_ref

ctrl = SE3Controller(QuadParams())
p_ref = np.array([0.0, 0.0, 1.0])

def tilt(deg, axis):
    axis = np.asarray(axis, dtype=float) / np.linalg.norm(axis)
    h = np.deg2rad(deg) / 2.0
    return quat_normalize(np.r_[np.cos(h), np.sin(h) * axis])

qI = np.array([1.0, 0.0, 0.0, 0.0])
z3 = np.zeros(3)
cases = [
    ("hover, zero error ", [0, 0, 1.0],       z3,             qI,                 z3,              0.0),
    ("10 cm x offset    ", [0.1, 0, 1.0],     z3,             qI,                 z3,              0.0),
    ("sinking 0.2 m/s   ", [0, 0, 1.0],       [0, 0, -0.2],   qI,                 z3,              0.0),
    ("10 deg roll tilt  ", [0, 0, 1.0],       z3,             tilt(10, [1, 0, 0]), z3,             0.0),
    ("tilt + body rate  ", [0, 0, 1.0],       z3,             tilt(10, [1, 0, 0]), [1.0, -0.5, 0.2], 0.0),
    ("everything at once", [0.1, -0.05, 0.9], [0.2, 0, 0.1],  tilt(15, [1, 1, 0]), [1.0, -2.0, 0.5], 0.3),
]

rows = []
for name, p, v, q, w, yaw in cases:
    st = QuadState(p=np.asarray(p, float), v=np.asarray(v, float), q=q, w=np.asarray(w, float))
    ref_fn = hover_ref(p_ref, yaw=yaw)
    f, tau = ctrl.compute(st, ref_fn, t=0.0)
    ref = ref_fn(0.0)
    rows.append(np.r_[st.p, st.v, st.R.ravel(), st.w, ref.pos, ref.vel, ref.acc, ref.yaw, f, tau])
    print(f"{name}  f = {f:8.6f} N   tau = [{tau[0]:+12.5e} {tau[1]:+12.5e} {tau[2]:+12.5e}] N m")

header = "p(3), v(3), R(9 row-major), w(3), ref_pos(3), ref_vel(3), ref_acc(3), ref_yaw, f, tau(3)"
np.savetxt("out/golden_se3.csv", np.array(rows), fmt="%.17g", delimiter=",", header=header)
print(f"\nwrote out/golden_se3.csv  ({len(rows)} rows x {rows[0].size} columns)")
EOF

Observe, and check against theory before moving on: hover thrust is f = mg = 0.033 × 9.81 = 0.323730 N with zero torque; the sinking case adds exactly kv·0.2 = 0.033·8·0.2 = 0.0528 N (f = 0.376530); the 10° roll tilt gives f = mg·cos 10° = 0.318812 N and tau_x = J_x·(−k_R·sin 10°) = 1.43e-5 × (−1000 × 0.173648) = −2.48317e-3 N·m. Every printed number should be explainable this way — golden vectors you cannot explain are not golden, they are merely recorded.

Experiment 2 — compile the C port and pass Gate 1. This writes the complete C file — the chapter’s core functions plus vector helpers and a harness main that replays the CSV — compiles it with your desktop compiler, and diffs C against Python:

mkdir -p out && cat > out/golden_check.c <<'EOF'
/* golden_check.c — the SE(3) core math in firmware-style C, checked on the
 * laptop against golden vectors exported from quadsim/controller.py.
 * Discipline: float only (the STM32F405 FPU is single-precision), no malloc,
 * no libc beyond math.h (stdio/stdlib are harness-only, not for firmware). */
#include <math.h>
#include <stdio.h>
#include <stdlib.h>
#include <string.h>

/* ---- vehicle constants: quadsim/params.py + controller.py defaults ---- */
#define MASS_KG   0.033f
#define GRAV      9.81f
static const float J_DIAG[3] = {1.43e-5f, 1.43e-5f, 2.89e-5f};
static const float KP[3] = {0.033f * 16.0f, 0.033f * 16.0f, 0.033f * 16.0f};
static const float KV[3] = {0.033f * 8.0f, 0.033f * 8.0f, 0.033f * 8.0f};
static const float KR[3] = {1000.0f, 1000.0f, 100.0f};
static const float KW[3] = {63.0f, 63.0f, 20.0f};

/* ---- tiny static-allocation linear algebra (R is row-major float[9]) ---- */
static void cross3(const float a[3], const float b[3], float out[3]) {
    out[0] = a[1] * b[2] - a[2] * b[1];
    out[1] = a[2] * b[0] - a[0] * b[2];
    out[2] = a[0] * b[1] - a[1] * b[0];
}
static float dot3(const float a[3], const float b[3]) {
    return a[0] * b[0] + a[1] * b[1] + a[2] * b[2];
}
static float norm3(const float a[3]) { return sqrtf(dot3(a, a)); }

/* M = A^T * B, all 3x3 row-major */
static void mat3_at_b(const float a[9], const float b[9], float m[9]) {
    for (int i = 0; i < 3; ++i)
        for (int j = 0; j < 3; ++j)
            m[3 * i + j] = a[i] * b[j] + a[3 + i] * b[3 + j] + a[6 + i] * b[6 + j];
}

/* ---- the three functions that ARE the controller ---- */

/* R_cmd = flat_to_rotation(F_des, yaw): b3 along F, b1 near the yaw heading */
static void flat_to_rotation(const float f_des[3], float yaw, float r_cmd[9]) {
    float b1c[3] = {cosf(yaw), sinf(yaw), 0.0f};
    float b3[3] = {0.0f, 0.0f, 1.0f};
    float b1[3], b2[3], c[3];
    float n = norm3(f_des);
    if (n >= 1e-6f) {
        b3[0] = f_des[0] / n; b3[1] = f_des[1] / n; b3[2] = f_des[2] / n;
    }
    cross3(b3, b1c, c);
    if (norm3(c) < 1e-6f) {              /* singular heading: rotate b1c 90 deg */
        b1c[0] = cosf(yaw + (float)M_PI / 2.0f);
        b1c[1] = sinf(yaw + (float)M_PI / 2.0f);
        cross3(b3, b1c, c);
    }
    n = norm3(c);
    b2[0] = c[0] / n; b2[1] = c[1] / n; b2[2] = c[2] / n;
    cross3(b2, b3, b1);
    for (int i = 0; i < 3; ++i) {        /* columns of R_cmd are b1, b2, b3 */
        r_cmd[3 * i + 0] = b1[i];
        r_cmd[3 * i + 1] = b2[i];
        r_cmd[3 * i + 2] = b3[i];
    }
}

/* e_R = 0.5 * vee(R_cmd^T R - R^T R_cmd); the argument is exactly skew,
 * so vee reads the three off-diagonal entries directly */
static void attitude_error(const float r_cmd[9], const float r[9], float e_r[3]) {
    float m[9];
    mat3_at_b(r_cmd, r, m);              /* m = R_cmd^T R; R^T R_cmd = m^T   */
    e_r[0] = 0.5f * (m[3 * 2 + 1] - m[3 * 1 + 2]);
    e_r[1] = 0.5f * (m[3 * 0 + 2] - m[3 * 2 + 0]);
    e_r[2] = 0.5f * (m[3 * 1 + 0] - m[3 * 0 + 1]);
}

/* One controller step, hover/setpoint case (w_ff = a_ff = 0 — see chapter):
 * inputs are the estimator's state and one reference sample; outputs f (N,
 * clamped >= 0) and body torque tau (N m). */
static void se3_feedback(const float p[3], const float v[3], const float r[9],
                         const float w[3], const float ref_pos[3],
                         const float ref_vel[3], const float ref_acc[3],
                         float ref_yaw, float *f_out, float tau_out[3]) {
    float f_des[3], r_cmd[9], e_r[3], jw[3], wxjw[3], re3[3];
    for (int i = 0; i < 3; ++i)
        f_des[i] = -KP[i] * (p[i] - ref_pos[i]) - KV[i] * (v[i] - ref_vel[i])
                 + MASS_KG * ref_acc[i] + (i == 2 ? MASS_KG * GRAV : 0.0f);
    re3[0] = r[2]; re3[1] = r[5]; re3[2] = r[8];   /* R e3 = third column */
    *f_out = dot3(f_des, re3);
    if (*f_out < 0.0f) *f_out = 0.0f;
    flat_to_rotation(f_des, ref_yaw, r_cmd);
    attitude_error(r_cmd, r, e_r);
    for (int i = 0; i < 3; ++i) jw[i] = J_DIAG[i] * w[i];
    cross3(w, jw, wxjw);                           /* gyroscopic term w x Jw */
    for (int i = 0; i < 3; ++i)                    /* e_w = w when w_ff = 0  */
        tau_out[i] = J_DIAG[i] * (-KR[i] * e_r[i] - KW[i] * w[i]) + wxjw[i];
}

/* ---- harness: replay out/golden_se3.csv, report worst disagreement ---- */
int main(void) {
    FILE *fp = fopen("out/golden_se3.csv", "r");
    if (!fp) { fprintf(stderr, "run the export snippet first\n"); return 1; }
    char line[4096];
    float worst = 0.0f;
    int row = 0;
    while (fgets(line, sizeof line, fp)) {
        if (line[0] == '#') continue;
        float x[32];
        char *tok = strtok(line, ",");
        for (int i = 0; i < 32 && tok; ++i, tok = strtok(NULL, ","))
            x[i] = strtof(tok, NULL);
        float f, tau[3];
        se3_feedback(&x[0], &x[3], &x[6], &x[15], &x[18], &x[21], &x[24],
                     x[27], &f, tau);
        float err = fabsf(f - x[28]);
        for (int i = 0; i < 3; ++i) {
            float e = fabsf(tau[i] - x[29 + i]);
            if (e > err) err = e;
        }
        printf("row %d   f = %10.6f N (python %10.6f)   max |C - python| = %.3g\n",
               ++row, f, x[28], err);
        if (err > worst) worst = err;
    }
    fclose(fp);
    printf("worst disagreement over all rows: %.3g  (%s)\n", worst,
           worst < 1e-5f ? "PASS < 1e-5" : "FAIL");
    return worst < 1e-5f ? 0 : 1;
}
EOF
cc -std=c99 -Wall -Wextra -O2 -o out/golden_check out/golden_check.c -lm && ./out/golden_check

Observe: all six rows agree to 2.98e-08 or better — that is one unit-in-the-last-place of a float near 0.32, i.e. the C port in single precision reproduces the double-precision Python controller to the limit float allows. That number is Gate 1’s signature: not “close”, but explainably at machine precision. (Note what the harness is: stimulus from one implementation, response diffed against another — an HDL testbench, wearing C.)

Experiment 3 — hand-derive one golden vector, then (break it) flip the attitude-error transpose. First the derivation, because a gate you cannot derive is a gate you cannot debug. Take row 4: R is a pure roll by θ = 10°, R_cmd = I up to the tilt-induced correction — but at zero position/velocity error F_des = mge3, so R_cmd is exactly I and e_R = 0.5·vee(R − Rᵀ). For a single-axis rotation, R − Rᵀ has exactly one off-diagonal pair, of magnitude 2 sin θ, and vee’s averaging halves it twice: e_R = [sin θ, 0, 0] = [0.173648, 0, 0]. Torque: tau_x = J_x(−k_R·sin θ − k_w·0) = 1.43e-5 × (−1000 × 0.173648) = −2.48317e-3 N·m — the golden CSV’s row 4 to the last digit. Also read the sign: positive roll produces negative roll torque. That sign is the entire stability story.

Now break it. In out/golden_check.c, swap the transpose order — change mat3_at_b(r_cmd, r, m) to mat3_at_b(r, r_cmd, m) (computing RᵀR_cmd first, i.e. e_R negated) — recompile, rerun:

row 1   f =   0.323730 N (python   0.323730)   max |C - python| = 2.98e-08
row 2   f =   0.323730 N (python   0.323730)   max |C - python| = 0.0046
row 3   f =   0.376530 N (python   0.376530)   max |C - python| = 2.98e-08
row 4   f =   0.318812 N (python   0.318812)   max |C - python| = 0.00497
row 5   f =   0.318812 N (python   0.318812)   max |C - python| = 0.00497
row 6   f =   0.314042 N (python   0.314042)   max |C - python| = 0.0115
worst disagreement over all rows: 0.0115  (FAIL)

Two lessons, both load-bearing. First: rows 1 and 3 still pass — zero attitude error hides the bug completely, which is why a golden set with only symmetric, level cases would wave this catastrophe through to the props. Second: read the failing magnitudes — each is exactly twice the J·kR·e_R contribution to that row’s torque (row 4: 2 × 2.48317e-3 = 4.97e-3), because flipping the transpose order negates e_R and only that term: the attitude-feedback torque points the wrong way. On the bench (Gate 3) this reads as “tilt left, left motors slow down”; in the air it is positive feedback and instant departure. This one-character bug is precisely the class of error the mapping table’s “attitude-error convention” row warns about when comparing against controller_mellinger.c — now you have seen its fingerprint at every gate below flight.

Experiment 4 — fieldwork: fill the mapping table’s Verify column. Open controller_mellinger.c on GitHub (src/modules/src/controller/ in bitcraze/crazyflie-firmware) and, for each row of the mapping table, write one sentence in the Verify column of your workbook copy: what you found, at which commit hash (record it — the file changes). Search the file for i_error and ki (the integral machinery, ch07 Homework 3), for the thrust computation and its scaling, and for how the attitude error is built — cross products of body axes, or the skew-symmetric form, and with which sign convention relative to Experiment 3’s derivation. Budget an hour; this table, verified against a pinned commit, is the design document for the real port.

## Homework

- The gates essay: Gate 1 passed at 2.98e-08, yet Gate 3 (props-off bench) can still fail. Name two distinct classes of bug that survive perfect golden vectors and are caught on the bench, and for each say what you would observe — the fingerprint, not the category.

Your answer:

- Integral action: write the C for adding a ki term to our F_des — the static accumulator, its per-tick update, and its clamp — and explain in two sentences why the clamp (anti-windup) is non-optional on hardware when it was not even worth writing in the sim. (Where does the accumulator go during the seconds the vehicle sits on the pad with the controller running?)

Your answer:

- Conventions: Experiment 3 showed e_R = [sin θ, 0, 0] for a single-axis tilt. Some implementations use the axis-angle error rot_log(RᵀR_cmd) instead (our quadsim/maths.py has rot_log), which gives [−θ, 0, 0] for the same case (mind the sign convention). Show the two agree to first order for small θ, compute both at θ = 90° and θ = 179°, and say which behavior you would rather have commanding torque during an aggressive recovery — then say why matching the golden vectors still beats “better” during a port.

Your answer:

- The time budget: count the floating-point operations in Experiment 2’s se3_feedback path (including flat_to_rotation and attitude_error — count a multiply-add as two), plus the sqrtf and trig calls. Assuming order-of-magnitude 1 FLOP/cycle on the M4F FPU at 168 MHz and tens of cycles per libm call, estimate the tick cost in microseconds and compare to the 2 ms budget. What fraction did the math cost, and what does that tell you about where 500 Hz firmware actually spends its time?

Your answer:

- Design Gate 2: the --log CSV from flight/fly_hover.py carries t, x, y, z, vbat — list every additional signal a full closed-loop replay of controller.py needs (read compute()’s state and reference usage), and name the cfclient log variables or log-block layout you would configure to capture them, within the constraint that log blocks have limited size and rate (flight/README.md §C.4 and §C.6).

Your answer:

- Failure drill: you pass Gates 1–3, fly FT-1.1 with your port, and the vehicle oscillates fast in roll immediately after takeoff and is landed by Ctrl-C. Using ch07’s latency and gain arguments plus this chapter’s gates, list your first three hypotheses in the order you would test them, each with a test that needs no flight (bench, log replay, or golden-vector variant).

Your answer:

## Hints

- Think about what golden vectors never exercise: everything between your function and the motors, and everything about time.

- On the pad, e_p’s z-component is a constant ~0.5 m for as long as the script waits — integrate that.

- Both errors are odd functions with slope ±1 at zero (mind each convention’s sign); they part company as θ → π. sin θ folds back after 90°; θ keeps growing. Which keeps pushing hardest when the vehicle is nearly inverted?

- You should land in single-digit microseconds — a fraction under 1% of the tick. The rest of the answer is in ch07’s list of what the firmware does 500 times a second that is not control math.

- compute() reads p, v, R, w and the reference’s pos, vel, acc, yaw — the state side maps to stateEstimate.* variables; the setpoint side you already have, because you commanded it.

- One hypothesis is Experiment 3 wearing flight clothes; one is a units/scaling row of the mapping table; one is ch07 Experiment 2’s cliff. The order follows “cheapest test first.”

## Further reading

- The crazyflie-firmware repository and documentation (github.com/bitcraze/crazyflie-firmware; docs on bitcraze.io) — the controller sources under src/modules/src/controller/, the controller interface header under src/modules/interface/, and the documentation chapters on the out-of-tree build and the app layer: the authoritative, current answers to every API question this chapter deliberately hedged.

- Lee, Leok, McClamroch, “Geometric Tracking Control of a Quadrotor UAV on SE(3),” CDC 2010 — the equations you just ported, with the stability proofs that say why Experiment 3’s sign is the difference between a controller and an anti-controller.

- Mellinger & Kumar, “Minimum Snap Trajectory Generation and Control for Quadrotors,” ICRA 2011 — the paper controller_mellinger.c is named for; read its controller section against your verified mapping table.

- docs/ARCHITECTURE.md §2 and docs/FLIGHT_TEST_PLAN.md — the control architecture of record (what the port must preserve) and the campaign your firmware re-enters at FT-1.1 (what the port must pass).

← Previous: Chapter 8 — Decentralized swarms · Course index — this is the final chapter of the course.

# Appendix A — Reading the plots: five checks for every flight

Every demo in this repo ends in the same four-panel figure (quadsim/viz.py’s plot_tracking), and every real flight in the test campaign will end in the same panels drawn from a log (flight/analyze_log.py --plot). That is deliberate: reading this figure is a skill you use from your first simulation to your last flight test. This page is the checklist. Run .venv/bin/python demos/demo_hover.py --fast and open out/hover_tracking.png to follow along with real numbers.

The four panels, top-left to bottom-right: position (solid = actual, dashed = reference), position error norm (one scalar: distance to the reference), total thrust (the effort, with the f_max limit dashed red), and body rates (the attitude loop’s activity).

## The five checks

1. Find reference vs actual — their gap is the whole story. Dashed is where the vehicle should be; solid is where it is. Everything else on the page explains the gap between them. Hover demo: the gap starts at 0.40 m (= √(0.23² + 0.23² + 0.23²), the three initial offsets combined) and ends below a millimeter. In a real-flight plot the “actual” line is itself an estimate — keep ch07’s grain of salt.

2. Overshoot and ringing — read the damping. Does the solid line cross the dashed one and come back? That is overshoot: underdamping. Does the error oscillate as it decays? Ringing. Our default gains place both loops at critical damping (ζ = 1, ch03), so healthy plots show monotone convergence — no crossing, no ripple. A brief, sharp spike in body rates at the start is normal (that’s the tilt that moves the vehicle, ~±5 rad/s in the hover demo); sustained oscillation in body rates means the attitude loop is fighting itself — ch03’s starve-kR experiment shows exactly that signature.

3. Settle time vs prediction — the design audit. Before looking, predict: a critically damped loop settles in roughly 4–6/ωₙ. Position loop: ωₙ = √(kp/m) = √16 = 4 rad/s → predicted 1.0–1.5 s. Measured by the demo: settle_time_s: 1.182. If the measured value is far slower than predicted, something is stealing authority — usually saturation (check 4) or a missing feedforward (ch03 Experiment 3).

4. Actuator vs limits — hunt for saturation. The thrust panel’s dashed red line is f_max = 4 × 0.16 = 0.64 N. The command touching that line means the controller asked for more than the motors have; tracking then degrades silently — no error message, just a growing gap in panel 1. The hover demo cruises near half throttle (T/W ≈ 2, ch02); the aggressive figure-8 demo is where this check earns its keep. On real logs the same check applies to battery sag: a “sagged” pack lowers the true ceiling ~25 % (docs/DESIGN.md §9).

5. Steady-state physics audit — do the numbers land where physics says? After settling: thrust must equal weight — m·g = 0.033 × 9.81 = 0.324 N — and the hover demo’s thrust trace flattens exactly there. Body rates must go to zero. Error must flatten at ~0 in simulation (steady_rms_m: 0.00074); in real flight a constant error offset is the estimator-bias fingerprint e_p = −(k_v/k_p)·b from ch07 Experiment 1 — a 0.1 m/s velocity bias parks the vehicle 50 mm off target, forever. Flat-but-not-zero is a diagnosis, not a mystery.

## The same panels, elsewhere

| Where | What changes |
| --- | --- |
| demo_minsnap.py, demo_figure8.py | Reference is a moving curve, not a point; error stays nonzero during the maneuver (RMS targets: 0.08 m / 0.10 m), and check 4 matters most at the corners. |
| demo_swarm.py, demo_decentralized.py | One vehicle’s panels per quad plus formation metrics; add ch05/ch08’s min-pairwise-distance check to the list. |
| flight/analyze_log.py --plot | Same layout from a real log. Expect centimeters where sim shows millimeters (Flow-deck reality, docs/ESTIMATION.md), and run the five checks in order — they are the post-flight analysis the flight-test cards ask for. |

Five checks, one habit: reference vs actual, damping, settle time vs prediction, saturation, physics audit. If all five pass, the flight was healthy; whichever one fails names the chapter that explains it.

Course index